| Subject Code A19PE1EC06 |  |  |  |  |  |  |  |  | A19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year II Semester Regular Examinations, July 2024

CPLD AND FPGA ARCHITECTURE

(ECE)

Time: 3 hours                                                                                 			    Max. Marks: 70

Answer ALL Questions from PART-A

Answer ONE question from each unit of PART-B.

PART-A

5X2=10M

|  | What is a configurable PAL? | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | List out the applications of CPLDs. | 2M | CO-2 | BL-1 |
|  | Distinguish FPGAs and CPLDs. | 2M | CO-2 | BL-2 |
|  | Differentiate SRAM and Anti-fuse programming technologies. | 2M | CO-3 | BL-2 |
|  | Define simulation & synthesis in FPGA design flow. | 2M | CO-4 | BL-1 |

PART-B

6X10=60M

| UNIT-I |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | a) Distinguish PLA, PAL. b) A combinational circuit is defined by the functions Y1(A,B,C)=Σ(1,4,5,7) , Y2(A,B,C)=Σ(0,2,3,6) Implement the circuit with a PAL. |  | 5M 5M |  | CO-1 CO-1 |  | BL-3 BL-4 |  |
| OR |  |  |  |  |  |  |  |  |
|  |  | a) Discuss about programmable logic arrays with an example. b) Design a 512 x 8 ROM using 128 x 8 RAMs. |  | 5M 5M |  | CO-1 CO-1 |  | BL-2 BL-4 |
| UNIT-II |  |  |  |  |  |  |  |  |
|  |  | Explain the Architecture of Altera Max 7000 series and it’s macro cell with neat diagrams. |  | 10M |  | CO-2 |  | BL-2 |
| OR |  |  |  |  |  |  |  |  |
|  |  | Describe Xilinx Cool Runner-II XCR3064XL CPLD architecture. |  | 10M |  | CO-2 |  | BL-3 |
| UNIT-III |  |  |  |  |  |  |  |  |
|  |  | a) Discuss about Configurable Logic Block (CLB) of FPGA. b) Write the advantages & disadvantages of FPGAs. |  | 5M 5M |  | CO-2 CO-2 |  | BL-2 BL-2 |
| OR |  |  |  |  |  |  |  |  |
|  |  | a) Implement the function Y=∑m(2,5,9,11,14) using FPGA. b) List out commercially available FPGAs |  | 5M 5M |  | CO-2 CO-2 |  | BL-4 BL-2 |
| UNIT-IV |  |  |  |  |  |  |  |  |
|  |  | Explain Xilinx XC4000 series architecture. |  | 10M |  | CO-2 |  | BL-2 |
| OR |  |  |  |  |  |  |  |  |
|  |  | Discuss about Actel ACT1& ACT2 architectures. |  | 10M |  | CO-2 |  | BL-2 |
| UNIT-V |  |  |  |  |  |  |  |  |
|  |  | a) Discuss FPGA design flow. b) Explain the partitioning in FPGA design process. |  | 5M 5M |  | CO-3 CO-3 |  | BL-2 BL-2 |
| OR |  |  |  |  |  |  |  |  |
|  |  | a) Describe the various steps in FPGA physical design cycle. b) Distinguish segmented and non-segmented models for routing. |  | 5M 5M |  | CO-3 CO-3 |  | BL-2 BL-3 |
| UNIT-VI |  |  |  |  |  |  |  |  |
|  |  | Design a MOD-16 counter using FPGA? |  | 10M |  | CO-4 |  | BL-3 |
| OR |  |  |  |  |  |  |  |  |
|  |  | a) Outline the design issues in FPGA. b) Discuss the design process of adder and accumulator with ACT architecture. |  | 5M 5M |  | CO-4 CO-4 |  | BL-3 BL-3 |

***