| Subject Code A19PC1ME15 |  |  |  |  |  |  |  |  | A19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year II Semester Regular Examinations, July 2024

DESIGN OF MACHINE ELEMENTS

(ME)

Time: 3 hours                                                                                 		      	     Max. Marks: 70

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

Note: Use of design data book is permitted

Assume any missing data.

PART-A

5X2=10M

|  | Lis out the steps involved in design of a machine element? | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | What are the causes of stress concentration? | 2M | CO-1 | BL-1 |
|  | What are the advantages of welded joints over riveted joints? | 2M | CO-2 | BL-1 |
|  | Briefly explain the design requirements of piston. | 2M | CO-3 | BL-1 |
|  | What is hydrodynamic lubrication? | 2M | CO-3 | BL-2 |

PART-B

6X10=60M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | The principal stresses induced at a point in a machine component made of steel 50C4 (Syt = 460 N/mm2) are as follows: σ1= 200N/mm2 σ 2 = 150 N/mm2 σ3 = 0 Calculate the factor of safety by (i) the maximum shear stress theory, and (ii) the distortion energy theory. | 10M | CO-1 | BL-3 |
| OR |  |  |  |  |
|  | - Explain the significance of preferred numbers. - What are the factors to be considered for the selection of materials for the design of machine elements. | 5M 5M | CO-1 CO-1 | BL-2 BL-2 |
| UNIT-II |  |  |  |  |
|  | A round shaft made of a brittle material and subjected to a bending moment of 15 N-m is shown below. The stress concentration factor at the fillet is 1.5 and the ultimate tensile strength of the shaft material is 200 N/mm2. Determine the diameter d, the magnitude of stress at the fillet and the factor of safety. | 10M | CO-1 | BL-3 |
| OR |  |  |  |  |
|  | A hot rolled steel shaft is subjected to a torsional moment that varies from +350 N-m to -115 N-m and an applied bending moment at a critical section varies from 445 N-m to 225 N-m.The shaft is of uniform cross section. Determine the required shaft diameter. The material has an ultimate strength of 550 MPa and yield strength of 410 MPa. Take the endurance limit as half the ultimate strength, factor of safety of 2, size factor of 0.85 and a surface finish factor of 0.62. | 10M | CO-2 | BL-3 |
| UNIT-III |  |  |  |  |
|  | A steel plate subjected to a force of 3 kN and fi xed to a vertical channel by means of four identical bolts. The bolts are made of plain carbon steel 45C8 (Syt = 380 N/mm2) and the factor of safety is 2. Determine the diameter of the shank. | 10M | CO-2 | BL-3 |
| OR |  |  |  |  |
|  | Two plates, each 5 mm thick, are connected by means of four rivets. The permissible stresses for rivets and plates in tension, shear and compression are 80, 60 and 120 N/mm2 respectively. Calculate, - diameter of the rivets; - width of the plate; and - efficiency of the joint. | 10M | CO-2 | BL-3 |
| UNIT-IV |  |  |  |  |
|  | Design a horizontal belt drive for a centrifugal blower, the blower is belt driven at 600 r.p.m. by a 15 KW, 1750 r.p.m. electric motor. The centre distance is twice the diameter of the larger pulley.The density of the belt material = 1500 kg/m 3 ; maximum allowable stress = 4 MPa; μ 1 = 0.5(motor pulley); μ 2 = 0.4 (blower pulley); peripheral velocity of the belt = 20 m/s. Determine the following: - Pulley diameters. - Belt length - Cross-sectional area of the belt - Minimum initial tension for operation without slip. | 10M | CO-4 | BL-3 |
| OR |  |  |  |  |
|  | Determine the dimensions of cross-section of the connecting rod, for a diesel engine with the following data: Cylinder bore = 100 mm. Length of connecting rod = 320 mm Maximum gas pressure = 2.45 MPa, Factor of safety against buckling failure = 5. | 10M | CO-3 | BL-3 |
| UNIT-V |  |  |  |  |
|  | A ball bearing with a dynamic load capacity of 22.8 kN is subjected to a radial load of 10 kN. Calculate (i) the expected life in million revolutions that 90% of the bearings will reach; (ii) the corresponding life in hours, if the shaft is rotating at 1450 rpm; and (iii) the life that 50% of the bearings will complete or exceed before fatigue failure. | 10M | CO-3 | BL-3 |
| OR |  |  |  |  |
|  | What are journal bearings? Give a classification of these bearings. | 10M | CO-3 | BL-2 |
| UNIT-VI |  |  |  |  |
|  | The pitch circle diameters of the pinion and gear are 100 and 300 mm respectively. The pinion is made of plain carbon steel 4OC8 (Sut = 600 N/mm2) while the gear is made of grey cast iron FG 300 (Sut = 300 N/mm2). The pinion receives 5 kW power at 500 rpm through its shaft. The service factor and factor of safety can be taken as 1.5 each. The face width of the gear can be taken as ten times that of the module. Assume that the velocity factor accounts for the dynamic load. Calculate (i) module; and (ii) the number of teeth on the pinion and gear. | 10M | CO-4 | BL-4 |
| OR |  |  |  |  |
|  | A pair of straight bevel gears has a velocity ratio of 2:1. The pitch circle diameter of the pinion is 80 mm at the large end of the tooth. 5 kW power is supplied to the pinion, which rotates at 800 rpm. The face width is 40 mm and the pressure angle is 20°. Calculate the tangential, radial and axial components of the resultant tooth force acting on the pinion. | 10M | CO-4 | BL-4 |

***