| Subject Code A19PC1CE13 |  |  |  |  |  |  |  |  | A19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year II Semester Regular Examinations, July 2024

DESIGN OF STEEL STRUCTURES

(CIVIL ENGINEERING)

Time: 3 hours                                                                               			           Max. Marks: 70

Answer ONE question in PART-A.

Answer any THREE questions from each unit in PART-B.

Note: Use of Design code IS 800: 2007, Steel Tables are allowed. Assume any data if necessary

PART-A

1X28=28M

|  | Design a built up column 10m long to carry factored axial load of 1080kN. The column is restrained in position but not in direction at both the ends. Provide single lacing system with tie-plate and bolted connections. The column has two channels placed back to back. Assume steel grade Fe410 and bolts of grade 4.6. Sketch the details of the column | 28M | CO-2 | BL-4 |
| --- | --- | --- | --- | --- |
|  | Design a gantry girder for EOT crane, by using the following data. Crane capacity = 250kN, Self Weight of crane excluding crab = 200kN, Self weight of crab, hook, motor = 40 kN, Minimum hook approach = 1.20m, C/C distance between the wheels = 2.80m, Span of the gantry girder = 6m, C/C spacing between gantry rails = 12m, Self weight of rail section = 250N/m. Diameter of crane wheels = 150mm, Self weight of gantry girder = 2kN/m, Assume suitable design data if required. Perform the necessary checks and check the deflection of girder. Use steel of grade Fe410, fy 250MPa | 28M | CO-4 | BL-4 |

PART-B

3X14=42M

|  | Two flats (Fe 410 grade steel), each 210mm x 8mm are to be jointed using 20mm diameter and 4.6 grade bolts to form a lap joint. The joint is supposed to transfer a factored load of 250 kN. Design the joint and determine suitable pitch for the bolts. Also determine the strength and efficiency of joint. Sketch the details of joint. Use steel of grade Fe410. | 14M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | Design a diagonal member of roof truss (tension member) to carry the factored tensile load of 350kN. The length of diagonal is 3m. The tension member is to be connected to a gusset plate 16mm thick and is bolted with one line of 20mm diameter bolts of grade 4.6. Neatly sketch the detailing aspects of connection system. Use steel grade Fe410 and fy 250MPa | 14M | CO-2 | BL-4 |
|  | Design a stanchion (compression member) 3.5 m long in a building subjected to a factored load of 550 kN. Both the ends of the member are effectively restrained in direction and position. Sketch the details of the column. Use steel of grade Fe410. | 14M | CO-2 | BL-4 |
|  | Design a laterally supported beam that carries UDL of 25kN/m over simply supported span of 6m. Apply all design checks for serviceability conditions. Neatly sketch detailing aspects. Use steel grade Fe410. | 14M | CO-3 | BL-4 |
|  | A girder is subjected to an UDL of 60 kN/m excluding its self weight over a span of 10m. Design the following components of thin web welded plate girder. i) Web plate ii) Flange plate iii) Flange to web connection. Sketch the detailing of component elements. Use steel grade Fe410. | 14M | CO-4 | BL-4 |

***