| Subject Code A19PC1EC09 |  |  |  |  |  |  |  |  | A19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year II Semester Regular Examinations, July 2024

DIGITAL SIGNAL PROCESSING

(EIE)

Time: 3 hours                                                                                		       Max. Marks: 70

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A

5X2=10M

|  | Define power and energy signals. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Draw the basic structure of radix 2 DIF FFT algorithm. | 2M | CO-1 | BL-1 |
|  | What is analog and digital frequency mapping in impulse invariant method? | 2M | CO-2 | BL-1 |
|  | How would you design an interpolator to increase the sampling rate of a signal from 10 kHz to 40 kHz? | 2M | CO-3 | BL-3 |
|  | Compare FWL effects in FIR and IIR digital filters. | 2M | CO-4 | BL-2 |

PART-B

6X10=60M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Determine the response of discrete time system given by difference equation y(n)=-0.8y(n-1)+x(n). when the input is step sequence and y(-1)=0. | 10M | CO-1 | BL-5 |
| OR |  |  |  |  |
|  | a) Derive the relation between Z transform and DFT. b) Define and derive expression of system function. | 5M 5M | CO-1 CO-1 | BL-2 BL-2 |
| UNIT-II |  |  |  |  |
|  | Find the discrete fourier transform of an eight point sequence x(n)={1,1,1,1,0,0,0,0} with a neat sketch using DIF-FFT radix 2 algorithm. | 10M | CO-1 | BL-5 |
| OR |  |  |  |  |
|  | a) Compare DIT-FFT and DIF-FFT algorithms. b) State and prove Periodic Convolution property of DFT. | 5M 5M | CO-1 CO-1 | BL-4 BL-4 |
| UNIT-III |  |  |  |  |
|  | Convert analog filter with transfer function (s + 0.1) / [(s + 0.1)2 + 9] into a digital IIR filter using impulse invariant transformation. | 10M | CO-2 | BL-5 |
| OR |  |  |  |  |
|  | a) Derive an expression for order of the Chebyshev analog prototype filter. b) Realize the discrete system y(n)=-0.1y(n-1)+0.2y(n-2)+3x(n)+3.6x(n-1)+0.6x(n-2) using Cascade form. | 5M 5M | CO-2 CO-2 | BL-5 BL-3 |
| UNIT-IV |  |  |  |  |
|  | Design an ideal high pass filter using Hamming window with the desired frequency response. Hdejω=1 forπ/4≤ω≤π0 otherwise Find h(n) for N=11. | 10M | CO-2 | BL-6 |
| OR |  |  |  |  |
|  | a) Discuss the characteristics of linear phase FIR filters. b) Compare Hamming and Blackman windowing techniques for design of digital FIR filters. | 4M 6M | CO-2 CO-2 | BL-2 BL-4 |
| UNIT-V |  |  |  |  |
|  | Describe the decimation process with a factor of `M '. Obtain necessary expression, sketch frequency response. Also discuss aliasing effect. | 10M | CO-3 | BL-5 |
| OR |  |  |  |  |
|  | a) Describe the need of multi-rate digital signal processing. b) What is aliasing? What is the need for anti- aliasing filter prior to down sampling? | 5M 5M | CO-3 CO-3 | BL-2 BL-2 |
| UNIT-VI |  |  |  |  |
|  | Describe the phenomenon of limit cycles in IIR digital filters. What causes these limit cycles, and how do they affect the filter's performance? | 10M | CO-4 | BL-4 |
| OR |  |  |  |  |
|  | a) Discuss the impact of round-off noise on the performance of IIR digital filters. How does the recursive nature of IIR filters amplify the effects of round-off noise? b) Explain the concept of overflow oscillations in the context of finite word length arithmetic in digital filters. How do overflow oscillations differ from limit cycles? | 5M 5M | CO-4 CO-4 | BL-4 BL-2 |

***