| Subject Code A19PC1EE09 |  |  |  |  |  |  |  |  | A19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year II Semester Regular Examinations, July 2024

ELECTRICAL DRIVES

(EEE)

Time: 3 hours                                                                                 		      	     Max. Marks: 70

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A

5X2=10M

|  | List the advantages of electric drives. | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | What is called continuous and discontinuous conduction? | 2M | CO-3 | BL-1 |
|  | Why self-commutated devices are preferred over thyristors for chopper circuits? | 2M | CO-3 | BL-3 |
|  | Define slip regulation of induction motor? | 2M | CO-2 | BL-2 |
|  | What distinguishes a Brushless DC motor (BLDC) from a traditional brushed DC motor in terms of construction? | 2M | CO-2 | BL-2 |

PART-B

6X10=60M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | a) Draw and describe the block diagram of a typical electric drive system in detail. b) Explain the speed-torque conventions in electric drives and their significance in motor performance. | 5M 5M | CO-2 CO-2 | BL-2 BL-2 |
| OR |  |  |  |  |
|  | a) Describe the four quadrants of operation for an electric drive and provide examples of each quadrant. b) What are the main components of load torque in an electric drive system? Explain each component briefly. | 5M 5M | CO-1 CO-1 | BL-2 BL-2 |
| UNIT-II |  |  |  |  |
|  | a) Explain the working principle of a single-phase semi-controlled converter connected to a DC separately excited motor. Draw the circuit diagram and discuss the advantages and limitations of using such converters. b) A 200 V, 15 A, 1000 rpm DC shunt motor has the armature resistance of 0.5 Ω. It is fed from a single phase fully controlled rectifier with an AC source voltage of 230 V, 50 Hz. Assuming continuous conduction. What will be the (i) motor speed for α=60° at twice the rated torque. (ii) firing angle for rated motor torque and 750 rpm. | 5M 5M | CO-3 CO-3 | BL-2 BL-3 |
| OR |  |  |  |  |
|  | Derive torque-speed characteristics of separately excited DC motor fed by 3-Phase fully controlled converter with circuit diagram, modes of operation, operational voltage, and current characteristics in detail. | 10M | CO-3 | BL-5 |
| UNIT-III |  |  |  |  |
|  | a) An industrial application demands motoring and braking operation. Suggest a suitable arrangement of chopper with DC motor meeting two quadrant operation of chopper control in a DC motor drive. Justify your suggestion with relevant proof. b) A 230 V, 960 rpm and 200 A separately excited DC motor has an armature resistance of 0.02 Ω. This motor is fed from a chopper which provides both motoring and breaking operations. The source has a voltage of 230 V, Assuming continuous condition. - Calculate duty ratio for motoring operation at rated torque and 350 rpm. - Calculate duty ratio for braking operation at rated torque and 350 rpm. | 5M 5M | CO-3 CO-3 | BL-3 BL-3 |
| OR |  |  |  |  |
|  | a) How do you perform dynamic braking for separately excited dc motor using chopper? Describe and derive necessary relationships. b) Explain the concept of regenerative braking in DC drives. Provide a detailed analysis of how energy is fed back to the power supply and the benefits of this braking method. | 5M 5M | CO-1 CO-1 | BL-3 BL-2 |
| UNIT-IV |  |  |  |  |
|  | Draw and explain the torque-speed curve of an induction motor during motoring, regenerative region and variation with changes in applied voltage. Illustrate with appropriate circuit diagrams and discuss the impact on motor performance. | 10M | CO-1 | BL-3 |
| OR |  |  |  |  |
|  | Describe flux weakening operation in induction motors. Explain the conditions under which flux weakening occurs and its impact on the motor’s performance, particularly at high speeds. | 10M | CO-1 | BL-5 |
| UNIT-V |  |  |  |  |
|  | a) Explain the principle of V/f (voltage/frequency) control for induction motors. Discuss how maintaining a constant V/f ratio helps in controlling the speed and torque of the motor. b) Draw the equivalent circuit of an induction motor and explain each component's significance. How does this equivalent circuit help in analyzing the motor's performance? | 5M 5M | CO-2 CO-2 | BL-3 BL-3 |
| OR |  |  |  |  |
|  | Explain the concept of slip power recovery in slip-ring induction motors. Describe the working principle and performance of slip power recovery scheme using static Kramer drive. | 10M | CO-3 | BL-3 |
| UNIT-VI |  |  |  |  |
|  | a) Explain the operation of VSI fed self-controlled synchronous motor drive. b) Draw and explain the block diagram of a closed loop synchronous motor drive fed from VSI. | 5M 5M | CO-3 CO-3 | BL-2 BL-2 |
| OR |  |  |  |  |
|  | a) Explain the principle of operation of a Permanent Magnet Synchronous Motor. Discuss how the interaction between the stator and rotor fields leads to synchronous rotation and the role of the permanent magnets. b) Explain the converter drive operation of BLDC motor with neat circuit diagram. | 5M 5M | CO-2 CO-2 | BL-2 BL-2 |

***