| Subject Code A19PC1EE08 |  |  |  |  |  |  |  |  | A19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year II Semester Regular Examinations, July 2024

ELECTRICAL MEASUREMENTS AND INSTRUMENTATION

(EEE)

Time: 3 hours                                                                                		       Max. Marks: 70

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A

5X2=10M

|  | V1 is (100±0.01) V and V2 is (50±0.05) V, when two voltmeters are connected in series calculate the uncertainty in measuring in voltage. | 2M | CO-1,2 | BL-2 |
| --- | --- | --- | --- | --- |
|  | List the advantages of moving iron instruments. | 2M | CO-1,2 | BL-1 |
|  | Draw the high voltage shearing bridge diagram. | 2M | CO-1 | BL-1 |
|  | List the various selection factors to be considered for Instrument transformer. | 2M | CO-4 | BL-1 |
|  | Define Poisson’s ratio and Gauge factor of Strain gauge. | 2M | CO-3 | BL-1 |

PART-B

6X10=60M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | a) Describe the various operating forces required for proper operation of analog indicating instruments and write its importance. | 5M | CO-1,2 | BL-2 |
|  | b) Explain about static characteristics of measuring instrument. | 5M | CO-1,2 | BL-2 |
| OR |  |  |  |  |
|  | a) A three-phase power is measured by two wattmeter method, one of the wattmeter’s reads 50W with an accuracy error of ±1.5% and the second wattmeter reading is 125 W with an accuracy error of ±0.5% of reading, if both the wattmeters are having a full-scale deflection of 150 W, then find the error in measurement of total power in the three-phase circuit | 5M | CO-1,2 | BL-2 |
|  | b) Describe in detail the different types of dynamic errors in a measurement system | 5M | CO-1,2 | BL-2 |
| UNIT-II |  |  |  |  |
|  | a) Explain the construction principle and operation of a PMMC instrument. | 5M | CO-1,2 | BL-2 |
|  | b) Discuss in detail Aryton shunt method used for extension range of ammeter with suitable assumptions. | 5M | CO-1,2 | BL-2 |
| OR |  |  |  |  |
|  | a) Describe the constructional details of the Electrodynamometer type power factor meter with a neat sketch. | 5M | CO-1,2 | BL-2 |
|  | b) An electro dynamic wattmeter is used to measure the power consumed by the load. The load voltage is 250v and the load current is 10A at a lagging power factor of 0.5. The wattmeter voltage circuit has a resistance of 2000 Ω and inductance 40 mH. Estimate the percentage error when the wattmeter voltage coil (moving coil) is connected directly across the load and on the supply side, the current coil resistance is 0.2 Ω.(Assume the supply frequency = 50 HZ) | 5M | CO-1,2 | BL-2 |
| UNIT-III |  |  |  |  |
|  | Derive the expression for an unknown resistance of Kelvin’s double bridge used for unknown resistance measurement. | 10M | CO-1,2 | BL-2 |
| OR |  |  |  |  |
|  | Describe the working of Anderson’s bridge for measurement of inductance & derive equations for balance. Why is this bridge suited for measurement of inductance of low Q coils? | 10M | CO-1,2 | BL-2 |
| UNIT-IV |  |  |  |  |
|  | A current transformer has a bar primary and 400 secondary winding turns. The secondary winding burden is an ammeter of resistance 1.4Ω and reactance 0.5Ω, the secondary winding has a resistance of 0.4Ω and reactance of 0.3Ω. The core requires an mmf of 200AT for magnetising and 80AT to supply core losses. (i) Find the primary winding current and ratio error when the ammeter in the secondary winding circuit indicates 5A. (4M) (ii) How many turns could be reduced in the secondary winding to make ratio error zero? (4M) ii) Draw the equality circuit. (2M) | 10M | CO-4 | BL-3 |
| OR |  |  |  |  |
|  | a) Define the term standardization and explain the working principle of Crompton potentiometer with a neat sketch and also list its applications. | 5M | CO-1,2 | BL-1 |
|  | b) Demonstrate the calibration process of an Ammeter using DC potentiometer. | 5M | CO-1,2 | BL-3 |
| UNIT-V |  |  |  |  |
|  | Describe in detail the working of Ramp type digital voltmeter with the help of neat block diagram and list its applications | 10M | CO-1,2 | BL-2 |
| OR |  |  |  |  |
|  | Explain the operation of Digital storage oscilloscope with a neat block diagram. | 10M | CO-1,2 | BL-2 |
| UNIT-VI |  |  |  |  |
|  | Define transducer and give the classification of transducers. | 10M | CO-3 | BL-1 |
| OR |  |  |  |  |
|  | a) Explain measurement working of LVDT instrument and list its application. | 5M | CO-3 | BL-2 |
|  | b) With neat block diagram explain the Data Acquisition Systems. | 5M | CO-3 | BL-2 |

***