| Subject Code A19PC1CE16 |  |  |  |  |  |  |  |  | A19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year II Semester Regular Examinations, July 2024

ESTIMATION AND COSTING

(CE)

Time: 3 hours                                                                                 			     Max. Marks: 70

Answer ALL Questions from PART-A

Answer ONE question from each unit of PART-B.

PART-A

5X2=10M

|  | Write the units of measurements for the following item of work i) Earthwork excavation, ii) DPC layer, iii) Flooring in upper floors, iv) Steel reinforcement. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Differentiate between mid-sectional and mean sectional area methods. | 2M | CO-2 | BL-2 |
|  | Determine the net area of internal plastering for a room with two openings measuring 1.2 x 2.1 m and 1.2 x 1.2 m each, using cement mortar (1:6) with a thickness of 12 mm. The room measures 5 m by 6 m from inner to inner. | 2M | CO-3 | BL-2 |
|  | Define the terms contract and tender. | 2M | CO-4 | BL-1 |
|  | Draw the tabular form of the detailed and abstract estimate methods. | 2M | CO-2 | BL-2 |

PART-B

6X10=60M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Prepare an approximate estimate of the building project for the following data: - Plinth area of building is 1200 sqm. - Plinth area rate Rs. 5000 per sqm - Cost of water supply @ 8 % of estimated cost of building. - Cost of Sanitary and electrical installations each @ 7.5 % of the cost of building. - Cost of roads and lawns @ 7 % of the building cost. - Cost of overhead and contingencies @ 10 % of the building cost. Determine the total cost of the building project. | 10M | CO-1 | BL-2 |
| OR |  |  |  |  |
|  | a) Describe the detailed specifications for the first-class brick masonry. b) List the types of estimates. Explain revised and supplementary estimates. | 5M 5M | CO-1 CO-1 | BL-2 BL-2 |
| UNIT-II |  |  |  |  |
|  | Utilize the long wall and short wall method to estimate the quantities from the given plan and section of a residential building. a.) Earth work in excavation b) Brick work in foundation and Superstructure with deductions (size of door – 1.2 x 2.0 m; window – 1.0x1.20 m) c) R.C.C Slab (M20 grade concrete) d) Flooring with tiles | 10M | CO-2 | BL-3 |
| OR |  |  |  |  |
|  | Calculate the cost per one cum of RCC Slab (1:1.5:3) with 3% steel by rate analysis. | 10M | CO-2 | BL-3 |
| UNIT-III |  |  |  |  |
|  | Explain in detail estimation of earthwork in irrigation channels for different cases with neat sketches. | 10M | CO-2 | BL-2 |
| OR |  |  |  |  |
|  | The formation width of road embankment is 12.0 m. The side slopes are 1:1.5. The depths along the centre line of road at 100.0 m intervals are 1.5, 1.4, 1.1, 0.9, 1.2, 1.3 and 1.0 m. It is required to calculate the quantity of earthwork by i) Prismoidal rule ii) Trapezoidal rule. | 10M | CO-2 | BL-3 |
| UNIT-IV |  |  |  |  |
|  | Prepare a bar bending schedule for RCC beam of clear span 4.0 m and cross section of beam 250 x 400 mm, the beam is reinforced with 2 nos 16 mm straight bars and 1 no 12 mm bent-up bars in tension zone, 2 nos 12 mm anchor bars in compression zone. The spacing of stirrups throughout the beam is constant (8 mm @ 180 mm c/c) from the given drawing. Steel in detail and RCC work shall be calculated separately. (# represents no. in figure) | 10M | CO-3 | BL-3 |
| OR |  |  |  |  |
|  | Calculate the bar bending schedule for footing and column: - Size of footing = 2000 x 1500 x 750 mm. - Reinforcement-12 mm diameter at 150 mm c/c main steel and 10 mm diameter at 120 mm c/c. - Clear cover – 50 mm at bottom and all around - Size of the column = 300 x 450 mm, reinforced with 6 no’s 20 mm dia. and 8 mm dia. lateral ties with 150 mmc/c. - Clear cover 40 mm, dowel bars are 12 mm. | 10M | CO-3 | BL-3 |
| UNIT-V |  |  |  |  |
|  | List the types of contracts. Explain any four types in detail. | 10M | CO-4 | BL-2 |
| OR |  |  |  |  |
|  | Define the term tender. Explain the procedure for preparation of tender document. | 10M | CO-4 | BL-2 |
| UNIT-VI |  |  |  |  |
|  | A property with a south-facing plot of land, 3.5 meters wide road, 20 meters of frontage, and 50 meters of depth is proposed to be developed. The front belt land may be taken up to 15 meters, with a fixed value of Rs. 700 per square meter for the front belt land. It is suggested that a five-story building with a total height of 20 meters above the ground be built, with three meters on the east, 1.5 meters on the west, and seven meters on the north. Assuming a construction cost of Rs. 1500 per month and an investment yield of 8.5% gross from the rentable rate. Determine the value of the property. | 10M | CO-3 | BL-3 |
| OR |  |  |  |  |
|  | Write short notes on i) Year’s purchase ii) Sinking fund iii) Depreciation iv) Straight line method. | 10M | CO-3 | BL-2 |

***