| Subject Code A19PC1ME13 |  |  |  |  |  |  |  |  | A19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year II Semester Regular Examinations, July 2024

HEAT TRANSFER

(Mechanical Engineering)

Time: 3 hours                                                                                		       Max. Marks: 70

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A

5X2=10M

|  | Define thermal diffusivity. What is its dimension? | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Define the efficiency and effectiveness of a fin. | 2M | CO-1 | BL-1 |
|  | Distinguish between natural and forced convection heat transfer. | 2M | CO-2 | BL-1 |
|  | Define the terms boiling and condensation. | 2M | CO-2 | BL-1 |
|  | Define NTU, specify its significance. | 2M | CO-4 | BL-1 |

PART-B

6X10=60M

| UNIT-I |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Derive the expression for general heat conduction in Cartesian coordinate system. | 10M |  | CO-1 | BL-3 |  |
| OR |  |  |  |  |  |  |
|  | a) State Fourier‘s law of heat conduction. Why is the negative sign used. b) What are initial and boundary conditions? | 5M 5M |  | CO-1 CO-1 | BL-3 BL-3 |  |
| UNIT-II |  |  |  |  |  |  |
|  | A plane wall 10 cm thick generates heat at the rate of 4×104 W/m3, when an electric current is passed through it. The convective heat transfer coefficient between each face of the wall and ambient air is 50 W/m2 K. Determine i). The surface temperature ii). The maximum temperature of the wall. | 10M |  | CO-1 | BL-4 |  |
| OR |  |  |  |  |  |  |
|  | a) Derive the expression critical radius of insulation in case of cylinder. b) A 40×40 cm copper slab 5mm thick at a uniform temperature of 2500C suddenly has its surface temperature lowered to 300C. Find the time at which the slab temperature becomes 900C. Take ρ = 9000 kg/m3, C =0.38 kj/kg K , k = 370 W/m K and h = 90 W/m2K. | 5M 5M |  | CO-1 CO-1 | BL-3 BL-3 |  |
| UNIT-III |  |  |  |  |  |  |
|  | Show by dimensional analysis that for forced convection heat transfer, Nusselt number is a function of Reynolds number and Prandtl number. | 10M | CO-2 |  |  | BL-4 |
| OR |  |  |  |  |  |  |
|  | Air at 100C and at a pressure of 100 kPa is flowing over a plate at a velocity of 3 m/sec. If the plate is 30 cm wide and a temperature of 600C. Calculate the following quantities at x = 0.3m 1. Boundary layer thickness 2.Thermal boundary layer thickness 3.The heat transfer from plate | 10M |  | CO-2 | BL-3 |  |
| UNIT-IV |  |  |  |  |  |  |
|  | Show the various regimes in pool boiling and discuss the heat transfer mechanism in each region in detail. | 10M |  | CO-2 | BL-3 |  |
| OR |  |  |  |  |  |  |
|  | Water is boiled at the rate of 30 kg/h in a copper pan, 30 cm in diameter, at atmospheric pressure. Estimate the critical heat flux? | 10M |  | CO-2 | BL-4 |  |
| UNIT-V |  |  |  |  |  |  |
|  | Assuming sun’s surface temperature as 5800 K, assuming sun as black surface, calculate (i) λmax (ii) ebλ at λ =λmax (iii) eb | 10M |  | CO-3 | BL-3 |  |
| OR |  |  |  |  |  |  |
|  | Two large gray parallel plates, separated by a small distance, have surface temperatures of 400 K and 300 K. If the emissivity’s of the surfaces are 0.8. Calculate the net radiation heat exchange rate in W/m2 between two plates. | 10M |  | CO-3 | BL-4 |  |
| UNIT-VI |  |  |  |  |  |  |
|  | - Sketch the temperature variation in parallel flow and counter flow heat exchanger. - Define heat exchanger effectiveness and explain its significance. | 5M 5M |  | CO-4 CO-4 | BL-3 BL-2 |  |
| OR |  |  |  |  |  |  |
|  | a) Classify heat exchangers. b) Water (Cp = 4180 J/kgK) enters a counter flow, double pipe heat exchanger at 150C, flowing at the rate of 1300 kg/h. It is heated by oil (Cp = 2000 J/kgK) flowing at the rate of 550 kg/h from the inlet temperature of 940C. For an area of 1 m2 and an overall heat transfer coefficient of 1075 W/m2 K, determine the total heat transfer and the outlet temperatures of water and oil. | 5M 5M |  | CO-4 CO-4 | BL-2 BL-3 |  |

***