| Subject Code A19PC1CE15 |  |  |  |  |  |  |  |  | A19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year II Semester Regular Examinations, July 2024

IRRIGATION ENGINEERING

(CE)

Time: 3 hours                                                                                 		      	     Max. Marks: 70

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A

5X2=10M

|  | Name different crop seasons in India. | 2M | CO1 | BL1 |
| --- | --- | --- | --- | --- |
|  | Define spoil bank and borrow pit. | 2M | CO1 | BL1 |
|  | Distinguish between cross regulator and head regulator. | 2M | CO2 | BL2 |
|  | Differentiate between weir and barrage. | 2M | CO3 | BL2 |
|  | What are the different types of spillway gates? | 2M | CO4 | BL1 |

PART-B

6X10=60M

| UNIT-I |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | a) Explain different methods of surface irrigation with neat sketches. b) A water course has a culturable command area of 2000 Ha. The intensity of irrigation for crop A is 40% and for B 35%, both crops being Rabi crops. Crop A and B has kor period of 25 days and 15 days respectively. Kor depth for crop A and B has 20 cm and 15 cm respectively. Determine the discharge required in the water course if the canal losses are 20%. | 6M 4M | CO1 CO2 | BL2 BL4 |
| OR |  |  |  |  |  |
|  | The base period, intensity of irrigation and duty of water for various crops under a canal system are given below. Determine the reservoir capacity if the culturable Command area is 4000 hectares, canal losses is 20% and reservoir losses are 10%. Crop; Base period (days); Duty at field (Ha/cumecs); Intensity of irrigation. Wheat; 120; 1800; 20%. Sugar cane; 360; 1600; 20%. Cotton; 180; 1400; 15%. Rice; 120; 800; 30%. Vegetable; 120; 700; 20%. | 10M | CO2 | BL4 |
| UNIT-II |  |  |  |  |  |
|  | Design an unlined irrigation canal to carry a discharge of 1.5m3/sec, assume soil rugosity coefficient (N) = 0.0225, critical velocity ratio (m) =1 and B/D = 5.0. | 10M | CO4 | BL3 |
|  | OR |  |  |  |
|  | a) Explain the causes, effects & remedial measures of water logging. b) Design an irrigation canal based on Lacey’s theory for the following data: Design discharge (Q) = 50 m 3 /sec, Silt factor (f) = 1.0, Side slope = 0.5H: 1V | 4M 6M | CO2 CO4 | BL2 BL3 |
| UNIT-III |  |  |  |  |  |
| 5. | Describe different types of cross drainage structures and explain their selection criteria. | 10M | CO3 | BL2 |
| OR |  |  |  |  |  |
| 6. | Explain the design principals of Trapezoidal Notch type fall with neat sketch. | 10M | CO3 | BL2 |
| UNIT-IV |  |  |  |  |  |
| 7. | Explain with neat sketches various component parts of Diversion head work. | 10M | CO3 | BL2 |
| OR |  |  |  |  |  |
| 8. | a) What are the various causes of failure of weirs and mention their remedies? b) The following figure shows the section of a hydraulic structure founded on sand. Determine the average hydraulic gradient. Find the uplift pressure at A, 5 m from the u/s end of the floor and also find the thickness of the floor at this point taking specific gravity of floor material as 2.5. | 4M 6M | CO3 CO3 | BL2 BL3 |
| UNIT-V |  |  |  |  |  |
| 9. | Obtain the profile of a low gravity dam for the following data. Top width = 6 m, Height of wave = 2 m, Maximum Water Level =150 m, Bed level = 100 m and Specific gravity of concrete = 2.4. Also find the normal stresses developed at the toe and heel of dam considering u/s water pressure and weight of dam only. Assume no uplift pressure. | 10M | CO4 | BL4 |
| OR |  |  |  |  |  |
| 10. | a) Explain and derive expressions for various forces acting on a gravity dam. b) Explain the classification of earthen dams with neat sketch. | 5M 5M | CO4 CO4 | BL2 BL2 |
| UNIT-VI |  |  |  |  |  |
| 11. | Define spillway. Explain its functionality and classify various type of spillways. | 10M | CO4 | BL2 |
| OR |  |  |  |  |  |
| 12. | a) Write the step by step procedure of finding the minimum storage capacity of a reservoir by mass curve of inflow for the given demand. b) Discuss about the selection of suitable site for reservoir. | 6M 4M | CO4 CO4 | BL2 BL2 |

***