| Subject Code A19PE1EE04 |  |  |  |  |  |  |  |  | A19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year II Semester Regular Examinations, July 2024

POWER SYSTEM OPERATION AND CONTROL

(EEE)

Time: 3 hours                                                                                 		      	     Max. Marks: 70

Answer ALL Questions from PART-A.

Answer ONE question from each unit of PART-B.

PART-A

5X2=10M

|  | Draw heat curve and mention its significance. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | What is the need of unit commitment. | 2M | CO-1 | BL-1 |
|  | Explain the role of ALFC and AVR loops in power system network? | 2M | CO-2 | BL-2 |
|  | List the merits of PI controller in load frequency control. | 2M | CO-3 | BL-2 |
|  | List different types of compensating equipment for transmission systems. | 2M | CO-4 | BL-2 |

PART-B

6X10=60M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Derive the condition for optimal loading of generators without considering transmission loss. | 10M | CO-1 | BL-3 |
| OR |  |  |  |  |
|  | a) Derive the expression for transmission loss formula. b) The incremental cost characteristics of the two units in a plant are IC1 = 0.1P1 + 8 Rs/MWh, IC2 = 0.15P2 + 3 Rs/MWh. If the total load is 100 MW, estimate the optimum load schedule. | 5M 5M | CO-1 CO-1 | BL-2 BL-3 |
| UNIT-II |  |  |  |  |
|  | Derive the coordination equation for the optimal scheduling of hydrothermal interconnected power plants. | 10M | CO-1 | BL-3 |
| OR |  |  |  |  |
|  | a) Explain the problem of unit commitment. Discuss the constraints in solving the unit commitment problem. b) With the help of a flow chart, explain the dynamic programming method in unit commitment. | 5M 5M | CO-1 CO-1 | BL-2 BL-4 |
| UNIT-III |  |  |  |  |
|  | a) Draw a neat sketch of a typical turbine speed-governing system and explain the function of each component in speed-governing system. b) A 250 MVA synchronous generator operates at 50 Hz under nominal loading conditions. If an additional load of 100 MW is suddenly removed, assuming a time lag of 0.72 sec in the speed governor system which delays the opening of steam valve to meet the additional demand, determine the frequency of the system before the steam flow begins to decrease. Take H = 7 MW-sec/MVA of generator capacity. | 5M 5M | CO-2 CO-2 | BL-2 BL-3 |
| OR |  |  |  |  |
|  | Develop the transfer function of generator load model. | 10M | CO-2 | BL-4 |
| UNIT-IV |  |  |  |  |
|  | What is single area system? With the help of block diagram deduce dynamic response for single area load frequency control system. | 10M | CO-3 | BL-4 |
| OR |  |  |  |  |
|  | a) Explain how the steady state frequency error is determined in a single area control system. b) Why is it essential to maintain constant frequency in power system network? | 5M 5M | CO-3 CO-3 | BL-3 BL-2 |
| UNIT-V |  |  |  |  |
|  | With all necessary equations develop block diagram representation of a two-area load frequency control system. | 10M | CO-3 | BL-4 |
| OR |  |  |  |  |
|  | a) Discuss the importance of combined load frequency control and economic dispatch control with a neat block diagram. b) Two control areas having the following characteristics Area-1: R1 = 0.01 p.u, B1 = 0.8 p.u, Base MVA = 1000 Area-2: R2 = 0.015 p.u, B2 = 0.9 p.u, Base MVA = 1000 A load change of 150 MW occurs in area -2. Find the tie line power deviation | 5M 5M | CO-3 CO-3 | BL-2 BL-3 |
| UNIT-VI |  |  |  |  |
|  | Explain the working of following FACTS devices along with their electrical circuit representation i) TCR and ii) TSC-TCR | 10M | CO-4 | BL-2 |
| OR |  |  |  |  |
|  | a) Explain the advantages and disadvantages of different types of compensating equipment for transmission systems. b) What do mean by compensation of a line? Discuss briefly different methods of compensation. | 5M 5M | CO-4 CO-4 | BL-2 BL-2 |

***