| Subject Code A19PC1EI08 |  |  |  |  |  |  |  |  | A19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year II Semester Regular Examinations, July 2024

PROCESS CONTROL AUTOMATION

(EIE)

Time: 3 hours                                                                                 		      	     Max. Marks: 70

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A

5X2=10M

|  | List a few devices to which PLC Input and Output modules are connected. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Draw the ladder logic for basic two input interlock system. | 2M | CO-2 | BL-2 |
|  | What is the purpose of MCR function. | 2M | CO-2 | BL-1 |
|  | Mention few applications of SCADA. | 2M | CO-3 | BL-1 |
|  | Write about the signals that leave the RTU. | 2M | CO-4 | BL-1 |

PART-B

6X10=60M

| UNIT-I |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | a) Discuss about PLC CPU Power Supply Module with relevant sketches. | 5M | CO-1 | BL-2 |
|  | b) Draw and explain the significance of PLC analog I/O modules. | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |  |
|  | a) Explain the architecture of PLC with neat diagram and mention any two its applications. | 5M | CO-1 | BL-2 |
|  | b) With reference to the PLC discrete input modules what type of field input devices are suitable for use with them? | 5M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |  |
|  | a) Compare ladder Logic diagram and Relay ladder diagram with examples. | 5M | CO-2 | BL-3 |
|  | b) Develop a ladder logic diagram for an alarm system. There are four hazard inputs to the alarm system that go on some operational malfunction occurs. The system operates as follows If one input is on, nothing happens If any two inputs are on, a red pilot light goes on If any three inputs are on, an alarm siren sounds If all four are on, the fire department is notified | 5M | CO-2 | BL-3 |
| OR |  |  |  |  |  |
|  | a) Develop a ladder logic programme for Instant Forward-Reverse Change circuit of a motor. | 5M | CO-2 | BL-3 |
|  | b) Develop a ladder logic diagram for the following Boolean expression L+M+N+(Q.R).R=S | 5M | CO-2 | BL-3 |
| UNIT-III |  |  |  |  |  |
|  | a) Develop ladder logic for counting the net number of parts on a conveyor. The number of parts going on the conveyor is counted by one proximity device’s count. The number leaving the conveyor is counted by a second proximity’s device’s count. The indicator will be ON when at least 6 of Part A and 8 of Part B have entered the conveyor. | 5M | CO-2 | BL-3 |
|  | b) Develop a Ladder Logic Diagram for a grinding operation on a metal part, the coolant flow on the part must be on for an interval before the grinding process starts. when the process circuit is turned on, the coolant motor (CM) is turned ON. Eight seconds later the grinding process starts. | 5M | CO-2 | BL-3 |
| OR |  |  |  |  |  |
| 6 | a) Develop a Ladder Logic Diagram for a wood sawing operation. The sawing operation takes 4.6 seconds. After the sawing takes place a blower to blow away the sawdust is to keep running until the operation is turned off. | 5M | CO-2 | BL-3 |
|  | b) Develop a ladder Logic programme for a paint spray which has to run for 25 seconds after a count of 15 from a sensor. | 5M | CO-2 | BL-3 |
| UNIT-IV |  |  |  |  |  |
| 7. | Develop a Ladder diagram for controlling of two axis Robot using PLC. | 10M | CO-2 | BL-3 |
| OR |  |  |  |  |  |
| 8. | Develop a Ladder diagram for 3- stage Air Conditioning system. | 10M | CO-2 | BL-3 |
| UNIT-V |  |  |  |  |  |
| 9. | Explain the elements of SCADA system including hardware and software in detail. | 10M | CO-3 | BL-3 |
| OR |  |  |  |  |  |
| 10. | Elaborate Safety Instrumented Systems. | 10M | CO-3 | BL-3 |
| UNIT-VI |  |  |  |  |  |
| 11. | An RTU can gather data in the form of a discrete switch position. This is used for alarm and status monitoring. What three other forms of field data can an RTU gather? Describe in detail. | 10M | CO-4 | BL-3 |
| OR |  |  |  |  |  |
| 12. | Elaborate the design principles for effective HMI’s, considering factors like visualization, alarm management, and operator interaction | 10M | CO-4 | BL-3 |

***