| Subject Code A19PE1ME04 |  |  |  |  |  |  |  |  | A19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year II Semester Regular Examinations, July 2024

REFRIGERATION AND AIR CONDITIONING

(ME)

Time: 3 hours                                                                                 		      	     Max. Marks: 70

Answer ALL Questions from PART-A.

Answer ONE question from each unit of PART-B.

PART-A

5X2=10M

|  | Define Ton of Refrigeration. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Draw T-s, and P-h diagrams for simple VCRS with Compressor inlet condition as dry saturated vapour. | 2M | CO-2 | BL-2 |
|  | What is the purpose of Generator in VARS? | 2M | CO-3 | BL-1 |
|  | Draw heating and humidification process in psychrometric coordinates. | 2M | CO-4 | BL-2 |
|  | Classify various Air conditioning systems. | 2M | CO-4 | BL-1 |

PART-B

6X10=60M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | a) Classify refrigerants. b) Explain Boot strap Air refrigeration system with neat sketch. | 5M 5M | CO-1 CO-1 | BL-2 BL-3 |
| OR |  |  |  |  |
|  | a) Explain Bell-Coleman cycle of refrigeration with neat sketch. b) What are the desirable properties of Refrigerants? | 5M 5M | CO-1 CO-1 | BL-3 BL-2 |
| UNIT-II |  |  |  |  |
|  | Explain the operation of Vapour compression refrigeration system and derive expression for C.O.P. | 10M | CO-2 | BL-3 |
| OR |  |  |  |  |
|  | a) Discuss the effect of sub cooling and super heating in VCRS. b) A vapour compression refrigerator uses methyl chloride (R-40) and operates between temperature limits of -10°C and 45°C. At entry to the compressor, the refrigerant is dry saturated and after compression it acquires a temperature of 60°C. Find the C.O.P. of the refrigerator. The relevant properties of methyl chloride are as follows | 5M 5M | CO-2 CO-2 | BL-2 BL-4 |
| UNIT-III |  |  |  |  |
|  | Classify compressors and compare single and multi stage compression. | 10M | CO-2 | BL-2 |
| OR |  |  |  |  |
|  | With the help of line diagram, explain the working of the flooded type of evaporator and write its advantages and Limitations. | 10M | CO-2 | BL-3 |
| UNIT-IV |  |  |  |  |
|  | Explain the Working of Li-Br VAR system. | 10M | CO-3 | BL-3 |
| OR |  |  |  |  |
|  | With a neat sketch explain the working principle of a steam jet refrigeration system. | 10M | CO-3 | BL-3 |
| UNIT-V |  |  |  |  |
|  | What is a Psychrometric chart and indicate the following processes in it. (a) Sensible heating (b) humidifying (c) Cooling and dehumidification (d) Adiabatic saturation and (e) chemical dehumidification | 10M | CO-4 | BL-2 |
| OR |  |  |  |  |
|  | The pressure and temperature of a mixture of dry air and water vapour are 736 mm of Hg and 210C. The dew point temperature of the mixture is 150C. Determine the following using steam tables. 1. Partial pressures of water vapour 2. Relative humidity 3. Specific humidity 4. Enthalpy of mixture per kg of dry air | 10M | CO-4 | BL-4 |
| UNIT-VI |  |  |  |  |
|  | What are the different types of fans used in air-conditioning systems? Discuss their applications and relative advantages and disadvantages. | 10M | CO-4 | BL-3 |
| OR |  |  |  |  |
|  | With a neat sketch explain the working of winter air conditioning system | 10M | CO-4 | BL-3 |

***