| Subject Code A19PC1AE09 |  |  |  |  |  |  |  |  | A19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year II Semester Regular Examinations, July 2024

VEHICLE DYNAMICS

(AE)

Time: 3 hours                                                                                 		      	     Max. Marks: 70

Answer ALL Questions from PART-A.

Answer ONE question from each unit of PART-B.

PART-A

5X2=10M

|  | Define lumped mass in the context of vehicle modelling. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Define traction-limited acceleration. | 2M | CO-1 | BL-1 |
|  | What is aerodynamic drag? | 2M | CO-1 | BL-1 |
|  | What is camber thrust? | 2M | CO-2 | BL-1 |
|  | Define suspension isolation. | 2M | CO-3 | BL-1 |

PART-B

6X10=60M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | - Explain the working principle of automatic transmission. - Derive axle load for load speed acceleration. | 5M 5M | CO-1 CO-1 | BL-2 BL-2 |
| OR |  |  |  |  |
|  | Find the traction-limited acceleration for the rear-drive passenger car with and without a locking differential on a surface of moderate friction level. Data needed as follows: Weights Front-2100 lb; CG height 21.0 in; Coefficient of friction 0.62; Final drive ratio 2.90; Roll stiffnesses Front -1150 ft-lb/deg; Rear-18501b Total-3950 lb;Wheelbase -108 in; Tread-59.0 in;Tire size-13.0 in; Rear -280 ft-lb/deg | 10M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |
|  | Calculate the braking coefficients and braking efficiency for a passenger car in 100 psi increments of application pressure up to 700 psi, given the following information: Wheelbase = 108.5 in; CGH = 20.5 in; Tire radius = 12.11 in;Weights: Wf= 2210 lb; Wr = 1864 lb; Total = 4074 lb;Front brake gain = 20 in-lb/psi; Rear brake gain = 14 in-lb/psi; Proportioning valve design = 290/0.3 | 10M | CO-1 | BL-3 |
| OR |  |  |  |  |
|  | - Discuss the different factors that influence a vehicle's braking performance. | 4M | CO-1 | BL-2 |
|  | - Discuss working principle of antilock braking system also mention its advantages. | 6M | CO-1 | BL-2 |
| UNIT-III |  |  |  |  |
|  | Evaluate the dynamics of tyre forces and moments with a neat sketch, and their impact on vehicle safety and performance. | 10M | CO-2 | BL-4 |
| OR |  |  |  |  |
|  | - Describe the performance of tyres on wet surfaces and ride properties of tyres. - Explain the cornering properties of tyre with neat sketches. | 4M 6M | CO-2 CO-2 | BL-2 BL-2 |
| UNIT-IV |  |  |  |  |
|  | - Explain about aerodynamic aids with neat diagram. - Describe the mechanics of air flow around a vehicle. | 4M 6M | CO-2 CO-2 | BL-2 BL-2 |
| OR |  |  |  |  |
|  | - Write the expression for Crosswind Sensitivity. - What are the key factors that influence Crosswind Sensitivity. | 5M 5M | CO-2 CO-2 | BL-2 BL-2 |
| UNIT-V |  |  |  |  |
|  | Describe how road roughness, tyre/wheel assembly, driveline excitation, and engine/transmission contribute to vehicle vibrations and noise. | 10M | CO-3 | BL-3 |
| OR |  |  |  |  |
|  | Derive pitch /bounce frequencies using suitable model. | 10M | CO-3 | BL-3 |
| UNIT-VI |  |  |  |  |
|  | Explain the dynamics of low-speed turning and high-speed cornering in vehicles, focusing on tyre cornering forces and the relevant cornering equations. | 10M | CO-3 | BL-3 |
| OR |  |  |  |  |
|  | How roll moment distribution, camber change, roll steer, lateral force compliance steer, and aligning torque influence a vehicle's behavior during cornering. | 10M | CO-3 | BL-3 |

***