| Subject Code A19PE1CE10 |  |  |  |  |  |  |  |  | A19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year II Semester Regular Examinations, July 2024

WATER SUPPLY ENGINEERING

(CE)

Time: 3 hours                                                                                 		      	     Max. Marks: 70

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A

5X2=10M

|  | List out the types of water demands. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Explain how an air release valve functions in a water distribution pipeline and why it is important. | 2M | CO-1 | BL-3 |
|  | Describe how the addition of coagulants helps in the removal of suspended particles during water treatment. | 2M | CO-2 | BL-2 |
|  | List two methods used for removing iron and manganese from water. | 2M | CO-2 | BL-2 |
|  | Explain how the use of alternative fuels, such as electricity or biofuels, can help in controlling air pollution from automobiles. | 2M | CO-4 | BL-2 |

PART-B

6X10=60M

| UNIT-I |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | a) Explain the importance and necessity of protected water supply systems. b) Describe the factors that influence the design period for water supply systems and discuss how water demand is estimated. | 5M 5M | CO-1 CO-1 | BL-2 BL-2 |
| OR |  |  |  |  |  |
|  | a) Define the term "hydrology" and explain its relevance in the context of water supply systems. Discuss the main hydrological concepts that are important for water resource management. b) Briefly discuss the water quality characteristics. | 5M 5M | CO-1 CO-1 | BL-2 BL-2 |
| UNIT-II |  |  |  |  |  |
|  | a) Explain the main functions of an intake structure in a water supply system and draw a labeled sketch of a typical intake structure b) Describe the factors that influence the selection of pipe materials for water transportation. | 5M 5M | CO-1 CO-1 | BL-2 BL-2 |
| OR |  |  |  |  |  |
|  | a) Define the term "pipe appurtenances". Provide examples of common pipe appurtenances and their functions. b) Explain the effects of corrosion on water supply infrastructure and describe the methods used for preventing corrosion. | 5M 5M | CO-1 CO-1 | BL-2 BL-3 |
| UNIT-III |  |  |  |  |  |
|  | a) Compare and contrast the working principles of gravity filters and pressure filters. b) Explain the working process of slow sand filters with a neat sketch. | 5M 5M | CO-2 CO-2 | BL-2 BL-2 |
| OR |  |  |  |  |  |
|  | a) Describe the process of sedimentation aided with coagulation in water treatment. b) A rectangular sedimentation tank is designed to treat a water flow of 20,000 m³/day. The tank is 50 meters long, 20 meters wide, and has an effective depth of 3 meters. The average settling velocity of the suspended particles is 0.5 mm/s. Determine the following: i) The detention time of the tank ii) The surface overflow rate (SOR) of the tank | 5M 5M | CO-2 CO-2 | BL-2 BL-3 |
| UNIT-IV |  |  |  |  |  |
|  | Describe the process of water softening and the methods used for removing hardness from water. | 10M | CO-2 | BL-2 |
| OR |  |  |  |  |  |
|  | Design a water treatment system for a small community with a population of 5,000 people. The groundwater source contains high levels of hardness (400 mg/L as CaCO3), iron (2 mg/L), and manganese (1 mg/L). Additionally, there is a concern about arsenic contamination (0.05 mg/L). The treated water should meet the following standards: - Hardness: < 100 mg/L as CaCO3 - Iron: < 0.3 mg/L - Manganese: < 0.05 mg/L - Arsenic: < 0.01 mg/L - Draw a process flow diagram and describe the proposed water treatment system, including the necessary unit operations and processes. (considering the quality parameters of the water which are mentioned above). - Calculate the required capacity of the treatment plant in cubic meters per day (m³/day), assuming a per capita water consumption of 150 liters per day. | 10M | CO-2 | BL-6 |
| UNIT-V |  |  |  |  |  |
|  | Describe the methods of water distribution in a network, including the advantages and disadvantages of each method. Discuss the factors that influence the selection of a suitable distribution method for a given area | 10M | CO-3 | BL-2 |
| OR |  |  |  |  |  |
|  | Calculate the head losses and the corrected flow in the various pipes of a distribution network shown in figure. The diameter and the lengths of the pipes used are given against each pipe. Make use of Hardy-Cross method with William Hazen’s formula. Compute the corrected flow after two corrections. | 10M | CO-3 | BL-6 |
| UNIT-VI |  |  |  |  |  |
|  | Describe the working principles and applications of particulate control devices, such as cyclone separators and electrostatic precipitators. Compare their effectiveness in removing particulate matter from air emissions | 10M | CO-4 | BL-2 |
| OR |  |  |  |  |  |
|  | a) Discuss the sources and effects of noise pollution. b) Discuss the common sources of indoor air pollution and their potential health effects on building occupants. | 5M 5M | CO-4 CO-4 | BL-3 BL-4 |

***