| Subject Code 19OE1AE03 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | R19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. IV Year I Semester Regular/Supplementary Examinations, November 2023

ELECTRIC, HYBRID AND FUEL CELL VEHICLES

(Open Elective)

Time: 3 hours                                                                                 		      		Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

PART-A

6X2=12M

|  | List out the advantages and disadvantages of EV vehicles. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | List the principles of electric propulsion systems in EV. | 2M | CO-2 | BL-1 |
|  | What is regenerative braking? | 2M | CO-3 | BL-1 |
|  | Write the working principle of a switched reluctance motor. | 2M | CO-3 | BL-1 |
|  | Define ultracapacitor? | 2M | CO-4 | BL-1 |
|  | List the types of fuel cells. | 2M | CO-4 | BL-1 |

PART-B

6X8=48M

| UNIT-1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | How does a traction motor operate in an electric or hybrid vehicle. Explain with neat diagram. | 8M | CO-1 | BL-2 |
| OR |  |  |  |  |  |
|  | Illustrate the electronic control system of an Electric Vehicle with a neat sketch. | 8M | CO-1 | BL-2 |
| UNIT-2 |  |  |  |  |  |
|  | Discuss in detail about series-parallel hybrid electric vehicles with a neat sketch. | 8M | CO-2 | BL-2 |
| OR |  |  |  |  |  |
|  | How are the torque and speed characteristics of electric motors in hybrid drivetrains selected to complement the internal combustion engine? | 8M | CO-2 | BL-2 |
| UNIT-3 |  |  |  |  |  |
|  | Describe the principle of operation and the construction of an induction motor with neat sketches. | 8M | CO-3 | BL-2 |
| OR |  |  |  |  |  |
|  | Explain the basic operation of a permanent magnet brushless DC motor and how does it differ from other types of motors? List out its advantages and disadvantages. | 8M | CO-3 | BL-3 |
| UNIT-4 |  |  |  |  |  |
|  | Elaborate the 4-quadrant operation of chopper DC motor drive in detail. | 8M | CO-3 | BL-3 |
| OR |  |  |  |  |  |
|  | Describe the structural components of a switched reluctance motor, including the rotor and stator. What role do phase currents and switching patterns play in torque generation? | 8M | CO-3 | BL-3 |
| UNIT-5 |  |  |  |  |  |
|  | Elaborate the Lead Acid Battery in detail with diagram and relevant equation. | 8M | CO-4 | BL-2 |
| OR |  |  |  |  |  |
|  | Illustrate in detail how the battery Management system function in an EV. Discuss its importance in modern electric vehicles. | 8M | CO-4 | BL-3 |
| UNIT-6 |  |  |  |  |  |
|  | Write notes on proton exchange membrane fuel cells and list the types of fuel cells used. | 8M | CO-4 | BL-3 |
| OR |  |  |  |  |  |
|  | Explain the working principle, drive train mechanism and key components of solar powered EV systems. | 8M | CO-4 | BL-2 |

***