| Subject Code 19PE1EC07 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | R19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. IV Year I Semester Regular/Supplementary Examinations, November 2023

FIBER OPTIC COMMUNICATION

(ECE)

Time: 3 hours                                                                                 		      	     Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

PART-A

6X2=12M

|  | List the applications of optical fiber communication. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | What is return loss? | 2M | CO-2 | BL-2 |
|  | What is the significance of Power bandwidth product in LED. | 2M | CO-3 | BL-4 |
|  | Define Detector response time. | 2M | CO-3 | BL-2 |
|  | List any two WDM standards. | 2M | CO-4 | BL-1 |
|  | What are the various multiplexing techniques? | 2M | CO-4 | BL-1 |

PART-B

6X8=48M

| UNIT-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | a) Label a neat block diagram and explain the fundamental blocks of optical fiber communication. b) List in detail about the linear scattering losses that occur in an optical fiber. | 4M 4M | CO-1 CO-1 | BL-1 BL-2 |
| OR |  |  |  |  |
|  | When the mean optical power launched into an 8𝑘𝑚 length of fiber is 120 µ𝑊, the mean optical power at the fiber output is 3 µ𝑊. Estimate: i) The overall signal attenuation or loss in decibels through the fiber assuming there are no connectors or splices. ii) The signal attenuation per kilometre for the fiber. iii) The overall signal attenuation for a 10 𝑘𝑚 optical link using the same fiber with splices at 1𝑘𝑚 intervals, each giving an attenuation of 1𝑑𝐵. iv) The numerical input and output power ratio in (iii). | 8M | CO-1 | BL-3 |
| UNIT-2 |  |  |  |  |
|  | a) What is Splicing in Optical fibers? List the different Splicing techniques. b) Illustrate the characteristics of Single mode and multimode fiber joints. | 4M 4M | CO-2 CO-2 | BL-2 BL-4 |
| OR |  |  |  |  |
|  | Explain connector types with their features and applications. | 8M | CO-2 | BL-1 |
| UNIT-3 |  |  |  |  |
|  | a) List the appropriate materials used for preparation of LED. b) Explain the Threshold conditions in Laser Diode operation. | 4M 4M | CO-3 CO-3 | BL-1 BL-2 |
| OR |  |  |  |  |
|  | a) With a diagram, explain surface emitters of LED structures. b) Explain about external quantum efficiency of Laser diode. | 4M 4M | CO-3 CO-3 | BL-2 BL-2 |
| UNIT-4 |  |  |  |  |
|  | a) Elucidate the physical principles of PIN Photodiode. b) Illustrate the effect of Temperature on Avalanche gain. | 4M 4M | CO-3 CO-3 | BL-1 BL-4 |
| OR |  |  |  |  |
|  | Describe the working principle of Avalanche photodiode with appropriate diagrams. | 8M | CO-3 | BL-4 |
| UNIT-5 |  |  |  |  |
|  | a) Explain the concept of Equilibrium Numerical Aperture. b) Outline the operation principles of WDM. | 4M 4M | CO-2 CO-4 | BL-4 BL-2 |
| OR |  |  |  |  |
|  | Derive the expression for power coupling from LED to step index and graded index fibers. | 8M | CO-2 | BL-3 |
| UNIT-6 |  |  |  |  |
|  | a) List the major components choices in Optical Link Design. b) Write short notes on Line coding in Optical links. | 4M 4M | CO-4 CO-4 | BL-2 BL-1 |
| OR |  |  |  |  |
|  | Explain how the Link power budget can be accessed for an Optical loss model. | 8M | CO-4 | BL-4 |

***