| Subject Code 19PE1CE11 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | R19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. IV Year I Semester Regular/Supplementary Examinations, November 2023

FOUNDATION ENGINEERING

(CE)

Time: 3 hours                                                                                 		      	     Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

PART-A

6X2=12M

|  | What are the various methods available for soil exploration? | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | What is the formula to determine the factor of safety in cohesionless dry soil? | 2M | CO-4 | BL-1 |
|  | Determine the coefficient of earth pressure at rest if the Poisson’s ratio of the soil is 0.3. | 2M | CO-3 | BL-3 |
|  | What are the requirements to be satisfied for retaining wall design? | 2M | CO-2 | BL-1 |
|  | List out the factors affecting the bearing capacity of shallow foundations. | 2M | CO-3 | BL-2 |
|  | Define negative skin friction. | 2M | CO-2 | BL-1 |

PART-B

6X8=48M

| UNIT-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Explain the direct, semi-direct and indirect methods of soil exploration. | 8M | CO-1 | BL-2 |
| OR |  |  |  |  |
|  | Explain seismic refraction method with neat sketch. | 8M | CO-1 | BL-2 |
| UNIT-2 |  |  |  |  |
|  | Derive the expression to determine the stability number of a slope. | 8M | CO-4 | BL-3 |
| OR |  |  |  |  |
|  | A cutting of depth 10 m is to made in soil which has c = 30 kN/m2 , γ = 19 kN/m3 and φ = 0. There is a hard stratum below the original soil surface at a depth of 12 m. Find the safe slope of cutting if the factor of safety is 1.5. For Df= 1.20; Sn= 0.143 for i = 300 ; Sn= 0.101 for i = 150 . | 8M | CO-4 | BL-2 |
| UNIT-3 |  |  |  |  |
|  | List out the assumptions in Rankine’s earth pressure theory. Derive an expression to determine the coefficient of active earth pressure. | 8M | CO-4 | BL-3 |
| OR |  |  |  |  |
|  | How will you calculate active earth pressure using Culmann’s method of construction? Provide neat sketch. | 8M | CO-4 | BL-1 |
| UNIT-4 |  |  |  |  |
|  | A retaining wall is 7 m high, with its back face smooth and vertical. It retains sand with its surface horizontal. Using Rankine’s theory, Determine active earth pressure at the base when the backfill is (a) dry (b) saturated and (c) submerged, with water table at the surface. Take γ = 18 kN/m3 and φ = 30o , γsat = 21 kN/m3. | 8M | CO-4 | BL-3 |
| OR |  |  |  |  |
|  | A retaining wall of height 10 m retaining a backfill soil having ϕ =35°, C =50 kN/m 2 γ = 20 kN/m 3, The backfill is carrying a uniform surcharge load of 45 kN/m 2. Calculate the total passive earth pressure, what is the point of application of resultant thrust. | 8M | CO-4 | BL-2 |
| UNIT-5 |  |  |  |  |
|  | Explain in detail the various types of shallow foundations and their applicability. | 8M | CO-2 | BL-2 |
| OR |  |  |  |  |
|  | Calculate the ultimate bearing capacity of a strip footing 2 m wide, with its base at a depth of 1.2 m, resting on dry sand and what is the difference in the ultimate bearing capacity when ground water table is present at a depth of 0.8 m from base of the footing. Take γ d = 15 kN/m3, γsat = 20 kN/m3 , ϕ’ = 39° , N c = 40, N q = 62 and N r =75 . | 8M | CO-2 | BL-2 |
| UNIT-6 |  |  |  |  |
|  | Explain the procedure to conduct the plate load test with a sketch and state its limitations. | 8M | CO-3 | BL-2 |
| OR |  |  |  |  |
|  | Design a square pile group to carry 600 kN in clay with an unconfined compression strength of 60 kN/m2. The piles are 30 cm diameter and 6 m long. Adhesion factor may be taken as 0.6. | 8M | CO-3 | BL-4 |

***