| Subject Code 19PE1ME12 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | R19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. IV Year I Semester Regular/Supplementary Examinations, November 2023

MECHANICS OF COMPOSITE MATERIALS

(ME)

Time: 3 hours                                                                                 		      	     Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

PART-A

6X2=12M

- List the biomedical applications of composite materials                                         	   CO-1 BL-2

- List four composite manufacturing defects.                                   			   CO-2 BL-2

- Name the precursors used in the manufacturing of carbon fibers.	    		    	   CO-2 BL-3

- List the advantages and disadvantages of Autoclave method.                        	               CO-3 BL-2

- What are the basic assumptions for Classical Laminate plate theory?		               CO-4 BL-2

- What do you mean by Warpage in Laminates?						   CO-4 BL-2

PART-B                                        6X8=48M

UNIT-1

- Define composite material. Give their classification. Also list the properties that can be enhanced by forming a composite material.			  					  CO-1 BL-2

OR

- Discuss the effect of fibre reinforcements on the overall performance of the composites. CO-1 BL-2

UNIT-2

- What is bag moulding? List the types of bag moulding and explain anyone with a neat sketch.                                                                                                            	     											  CO-1 BL 4

OR

- The fibers of a unidirectionally reinforced glass fiber/epoxy composite are continuous and 60% by volume. The tensile strength of glass fibers is 1 Gpa and the Young’s modulus is 70 Mpa. The tensile strength of the epoxy matrix is 60 MPa and its young’s modulus is 3 Gpa. Compute the Young’s modulus and tensile strength of the composite in the longitudinal direction.		 CO-1 BL 4

UNIT-3

- Discuss Polymer Infiltration and Pyrolysis (PIP) and Liquid Silicon Infiltration (LSI) process for manufacturing ceramic matrix composites and list the applications of Ceramic matrix composites.

CO-2 BL 3

OR

- With neat sketches, explain hot isostatic pressing used for fabrication of MMC’s.                                                                                                                                                                                                                                                  												CO-2 BL 3

UNIT-4

- With neat sketches explain: (i) Pultrusion (ii) Filament Winding.		  	CO-2 BL 2

OR

- Discuss the methods of characterization for a unidirectional lamina. 			CO-2 BL 2

UNIT-5

- Discuss the basic assumptions and Strain-displacement relations for elastic behavior of laminates.				              								CO-3 BL 4

OR

- Determine the stiffness matrices [A], [B] and [D] for a [45/-45]S  symmetric laminate. The properties for the fiber are Ef = 220 Gpa, gf = 0.2, Vf = 0.6. and for the matrix are Em = 220 Gpa, gm = 0.35. Each lamina is 6 mm thick.                             							  CO-3 BL 4

UNIT-6

- Discuss (i) Hygrothermal failure of composite materials (ii) Truncated maximum strain criterion. 													CO-4 BL- 2                         OR

- Determine the maximum value of a > 0 if stresses of sx = 3a,  sy = 2a,  txy = 5a, are applied to a 600 lamina of graphite/epoxy. The material properties of this lamina are given as follows;

E1 = 181 Gpa, E2 = 10.3 Gpa n12 = 0.28, Vf = 0.7, G12 = 7.17 Gpa, X=1500 Mpa, X’=1500 Mpa, Y = 40 Mpa, Y’ = 246 Mpa, and S= 68 Mpa. Use the following theories of failure;

- Maximum stress theory (ii) Maximum strain theory. 				  CO-4 BL- 2