| Subject Code 19PC1EC13 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | R19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. IV Year I Semester Regular/Supplementary Examinations, November 2023

MICROWAVE ENGINEERING

(ECE)

Time: 3 hours                                                                                 		      		Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

PART-A

6X2=12M

|  | Define the phase and group velocities | 2M | CO1 | BL1 |
| --- | --- | --- | --- | --- |
|  | What is a phase shifter? | 2M | CO2 | BL2 |
|  | What is the significance of scattering parameters? | 2M | CO2 | BL2 |
|  | What are the limitations of conventional tubes? | 2M | CO3 | BL3 |
|  | List the Avalanche Transit time Devices? | 2M | CO4 | BL2 |
|  | What is the use of Bolometer? | 2M | CO5 | BL3 |

PART-B

6X8=48M

| UNIT-1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | a)Discuss how the microwave spectrum is categorized into different bands. b)A waveguide having dimensions a = 5 cm, b = 2 cm. The signal applied to waveguide is10GHz. Determine the modes that are propagating in the waveguide. | 4M 4M | CO1 CO1 | BL2 BL3 |
| OR |  |  |  |  |  |
|  | Derive the expressions for the field components due to TE waves in rectangular wave guide. | 8M | CO1 | BL3 |
| UNIT-2 |  |  |  |  |  |
|  | a)Explain the coupling mechanism of waveguide. b)Calculate resonant frequency of rectangular cavity filled with dielectric with €r=4 and having dimensions a=5cm b=4cm and d=15cm. | 4M 4M | CO2 CO2 | BL2 BL3 |
| OR |  |  |  |  |  |
|  | Draw the diagram of dielectric phase shifter and discuss the working mechanism. | 8M | CO2 | BL4 |
| UNIT-3 |  |  |  |  |  |
|  | Explain following terms (a) Gyrator (b) Isolator | 4M 4M | CO2 CO2 | BL2 BL2 |
| OR |  |  |  |  |  |
|  | Explain the operation of a Magic Tee with a neat sketch and obtain its S matrix. | 8M | CO1 | BL3 |
| UNIT-4 |  |  |  |  |  |
|  | a)Write short notes on bunching process in a two-cavity klystron amplifier. b)Explain the possibility of oscillations in a TWT amplifier. | 4M 4M | CO3 CO3 | BL2 BL2 |
| OR |  |  |  |  |  |
|  | Describe with a neat sketch the constructional details and principle of operation of a reflex klystron tube. | 8M | CO3 | BL2 |
| UNIT-5 |  |  |  |  |  |
|  | a)Explain the V-I characteristics of a Gunn diode. b)An n-type GaAs Gunn diode has following parameters: Electron drift velocity: vd = 2.5 × 105 m/s. Negative electron mobility: µn = 0.015 m2 / v. s. Relative dielectric constant: €r = 13.1. Determine the criterion for classifying the modes of operation. | 4M 4M | CO2 CO2 | BL2 BL3 |
| OR |  |  |  |  |  |
|  | Draw and explain the construction of IMPATT and IMPATTdiodes | 8M | CO3 | BL4 |
| UNIT-6 |  |  |  |  |  |
|  | With the help of a neat sketch, briefly explain the functions of different blocks of a microwave bench. | 8M | CO3 | BL4 |
| OR |  |  |  |  |  |
|  | a)Explain the double minima method of measuring VSWR and different possible errors that will affect VSWR measurements? b) Write short notes on Network Analyzer. | 4M 4M | CO4 CO4 | BL2 BL2 |