| Subject Code 19PC1EC30 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | R19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. IV Year I Semester Regular/Supplementary Examinations, November 2023

PRINCIPLES OF DIGITAL SIGNAL PROCESSING

(EEE)

Time: 3 hours                                                                                 		      		Max. Marks: 60

Answer ALL questions in PART-A

Answer any ONE question from each unit in PART-B

All Questions Carry Equal Marks

***

PART-A

6X2=12M

|  | Determine whether or not the following signal is periodic. If periodic specify its fundamental period. xn =cos(0.5n+30°) | 2M | CO-1 | BL-3 |
| --- | --- | --- | --- | --- |
|  | Identify the number of multiplications and additions needed in the calculation of DFT using FFT algorithm with 32-point sequence. | 2M | CO-2 | BL-3 |
|  | Sketch a sample ROC for -anu(-n-1) | 2M | CO-2 | BL-3 |
|  | What is the condition that must be satisfied for a FIR filter to have linear phase characteristic? | 2M | CO-3 | BL-2 |
|  | Distinguish IIR and FIR filters. | 2M | CO-3 | BL-2 |
|  | Define pipelining in Digital Signal Processor. | 2M | CO-4 | BL-1 |

PART-B

6X8=48M

| UNIT-1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Identify whether the following systems are time invariant, linear and casual. i yt = -3t2tx(τ)dτ ii yt = ddtx(t) | 8M | CO-1 | BL-3 |
| OR |  |  |  |  |  |
|  | a) Obtain the following discrete time sequences by performing basic operations on unit step and unit impulse signals using Graphical Method. (i) u(2n+2) (ii) δ(3n-9) b) Examine whether the following systems are stable or not. i yn =n x(n) ii yn = ex(n) | 4M 4M | CO-1 CO-1 | BL-5 BL-5 |
| UNIT-2 |  |  |  |  |  |
|  | Perform the circular convolution of the following two sequences using DFT, x1(n) = { 1, 2, 2} and x2(n) = { 1, 2, 3, 4}. | 8M | CO-2 | BL-5 |
| OR |  |  |  |  |  |
|  | Determine the 8-point DFT of the given data sequence x(n) = {−1, 2,−3, 4, 9,−20, 12, 6} using DIT-FFT algorithm. | 8M | CO-2 | BL-5 |
| UNIT-3 |  |  |  |  |  |
|  | a) State and prove the following Z-Transform properties. i) Linearity ii) Shifting in time domain b) Sketch the ROC in Z plane by determining the Z transform of xn= 12nun+ 2n u(n) | 4M 4M | CO-2 CO-2 | BL-3 BL-3 |
| OR |  |  |  |  |  |
|  | Determine the causal signal x(n) having the Z-transform X(Z) using partial fraction method. XZ= 1(1+ Z-1) (1- Z-1)2 | 8M | CO-2 | BL-5 |
| UNIT-4 |  |  |  |  |  |
|  | Apply Bilinear transformation to convert the following analog filter with transfer function HA(s) to a digital filter where, HAs= s+0.2(s+0.2)2+16 | 8M | CO-3 | BL-5 |
| OR |  |  |  |  |  |
|  | Design an IIR Butterworth LPF for the following specifications. Assume T = 1 s. Passband : 0.8 ≤Hejω≤1 ; ω≤0.2 πStopband : Hejω≤0.2 ; 0.6π≤ω≤ π | 8M | CO-3 | BL-5 |
| UNIT-5 |  |  |  |  |  |
|  | Design an FIR LPF using a rectangular window with a pass band gain of unity, cut-off frequency of 0.4 π.Take the length of the impulse response as 7. | 8M | CO-3 | BL-5 |
| OR |  |  |  |  |  |
|  | Design an linear phase FIR LPF using Hamming window with the following desired frequency response: Hdejω =e-j2ω ;0 ≤¦ω¦≤π40 ; π4<¦ω¦≤π | 8M | CO-3 | BL-5 |
| UNIT-6 |  |  |  |  |  |
|  | Interpret the Digital Signal Processor hardware units. | 8M | CO-4 | BL-2 |
| OR |  |  |  |  |  |
|  | Illustrate the generation of PWM signals Using DSP Processor. | 8M | CO-4 | BL-2 |

***