| Subject Code 19PC1EE10 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | R19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. IV Year I Semester Regular/Supplementary Examinations, November 2023

SWITCHGEAR AND PROTECTION

(EEE)

Time: 3 hours                                                                                 		      		Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

PART-A

6X2=12M

|  | Define restriking voltage and recovery voltage | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | What is an impedance relay | 2M | CO-2 | BL-2 |
|  | Buchholz relay Protection, explain briefly | 2M | CO-3 | BL- 3 |
|  | State the importance of bus bar protection | 2M | CO-4 | BL-2 |
|  | Write the application of PMUs in power system protection | 2M | CO-5 | BL-4 |
|  | What is the need for AI in power systems | 2M | CO-5 | BL-3 |

PART-B

6X8=48M

| UNIT-1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | a) Explain the functions of basic elements of protection system b) Compare Fuse and Circuit breaker. | 4M 4M | CO-1 CO-1 | BL-1 BL-2 |
| OR |  |  |  |  |  |
|  | Explain SF6 circuit breaker with neat schematic diagram. | 8M | CO-1 | BL-3 |
| UNIT-2 |  |  |  |  |  |
|  | a) Derive the equation of torque produced in induction disc relay. b) Emphasize how a directional relay is used in power system protection | 4M 4M | CO-1 CO-1 | BL-2 BL-2 |
| OR |  |  |  |  |  |
|  | What is an impedance relay? Discuss its principle of operation. Show its characteristics on R-X diagram. What are the merits of this relay? | 8M | CO-2 | BL-3 |
| UNIT-3 |  |  |  |  |  |
|  | a) With neat diagram explain simple differential protection. b) What is Buchholz relay? Which equipment is protected by it? | 4M 4M | CO-2 CO-2 | BL-2 BL-2 |
| OR |  |  |  |  |  |
|  | A three phase transformer of 220/11000 line volts is connected in star/delta. The protective transformers on 220V side have a current ratio of 600/5. What should be ratio of current transformers on 11000V side? Draw the circuit also? | 8M | CO-2 | BL-3 |
| UNIT-4 |  |  |  |  |  |
|  | a) Explain directional over current static relays b) Define Amplitude Comparator and Phase Comparator | 4M 4M | CO-3 CO-3 | BL-2 BL-2 |
| OR |  |  |  |  |  |
|  | Compare the duality between amplitude and phase comparator. | 8M | CO-3 | BL-2 |
| UNIT-5 |  |  |  |  |  |
|  | a) With neat block diagram explain the working principle of numerical relay. b) explain briefly about digital relaying algorithm | 4M 4M | CO-4 CO-4 | BL-3 BL-3 |
| OR |  |  |  |  |  |
|  | Explain the concept of PMUs and its application in power system. | 8M | CO-4 | BL-3 |
| UNIT-6 |  |  |  |  |  |
|  | a) Explain about Artificial Neural Network Technique. b) What is the application ANN to over current protection | 4M 4M | CO-5 CO-5 | BL-4 BL-5 |
| OR |  |  |  |  |  |
|  | Analyze the power system by using ANN in power transformer and alternator protection. | 8M | CO-5 | BL-5 |