| Subject Code 19PE1ME18 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | R19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. IV Year I Semester Regular/Supplementary Examinations, November 2023

THEORY OF METAL CUTTING

(ME)

Time: 3 hours                                                                                 		      	    Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

PART-A

6X2=12M

|  | Write the expression for shear angle with respect to and rake angle and chip thickness coefficient? | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | What is crater wear? | 2M | CO-2 | BL-1 |
|  | Effect of nose radius on tool life and surface finish | 2M | CO-2 | BL-1 |
|  | State the three types of high speed tools | 2M | CO-3 | BL-1 |
|  | Importance of green cutting fluids | 2M | CO-3 | BL-1 |
|  | Write the expression for tool grinding cost per piece. | 2M | CO-4 | BL-2 |

PART-B

6X8=48M

| UNIT-1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | a) Distinguish between orthogonal and oblique cutting. b) State the conditions favourable to produce continuous chips. | 4M 4M | CO-1 CO-1 | BL-2 BL-1 |
| OR |  |  |  |  |  |
|  | The following data relate to an orthogonal turning process: Chip thickness = 0.62 mm Feed = 0.2 mm/rev Rake angle = 15ᴼ (i) Calculate cutting ratio and chip reduction coefficient. (ii) Calculate shear angle (iii) Calculate the dynamic shear strain involved in the deformation process. | 8M | CO-1 | BL-3 |
| UNIT-2 |  |  |  |  |  |
|  | With a neat sketch explain the working principle of piezoelectric lathe tool dynamometer. | 8M | CO-1 | BL-2 |
| OR |  |  |  |  |  |
|  | What are the factors that affect tool life? Briefly describe their influence. | 8M | CO-3 | BL-2 |
| UNIT-3 |  |  |  |  |  |
|  | a) Distinguish between single point and multi point cutting tools b) Sketch the nomenclature of tool in ASA System | 4M 4M | CO-2 CO-2 | BL-2 BL-3 |
| OR |  |  |  |  |  |
|  | Sketch the plain milling cutter and label its nomenclature. | 8M | CO-2 | BL-3 |
| UNIT-4 |  |  |  |  |  |
|  | a) How can you choose tool materials for machining hard materials? b) Elaborate the properties needed for tool materials? | 4M 4M | CO-2 CO-2 | BL-1 BL-2 |
| OR |  |  |  |  |  |
|  | Name various cutting tool materials. Briefly describe one important tool materials along with its characteristics and usability. | 8M | CO-2 | BL-2 |
| UNIT-5 |  |  |  |  |  |
|  | a) What are the sources of heat generation in machining? b) Draw a sketch to show heat distribution to various elements during metal cutting | 4M 4M | CO-3 CO-3 | BL-1 BL-3 |
| OR |  |  |  |  |  |
|  | What is the importance of cutting fluids in machining? Discuss the effect of cutting fluids in terms of environmental aspects. | 8M | CO-3 | BL-2 |
| UNIT-6 |  |  |  |  |  |
|  | What do you understand by economics of machining? How do you evaluate machining cost? | 4M | CO-4 | BL-1 |
| OR |  |  |  |  |  |
|  | Derive the Expression for tool life for maximum production rate. | 8M | CO-4 | BL-3 |

***