| Subject Code 19PE1CE18 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | R19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. IV Year I Semester Regular/Supplementary Examinations, November 2023

TRAFFIC ENGINEERING

(CE)

Time: 3 hours                                                                                 		      	      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

PART-A

6X2=12M

|  | Mention any two uses of speed studies. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | List the methods of conducting speed studies. | 2M | CO-1 | BL-1 |
|  | Mention some of the important factors that affect the practical capacity of a traffic lane. | 2M | CO-2 | BL-1 |
|  | List different types of off- street parking. | 2M | CO-3 | BL-1 |
|  | Define offset. | 2M | CO-4 | BL-1 |
|  | List major air pollutants from highway traffic. | 2M | CO-4 | BL-1 |

PART-B

6X8=48M

| UNIT-1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Explain different methods for conducting volume studies in detail. | 8M | CO-1 | BL-2 |
| OR |  |  |  |  |  |
|  | Assuming the linear density relationship, the mean free speed is observed to be 60kmph near zero density and the corresponding jam density is 140veh / km. - Draw V-K, V-Q and Q-K diagram indicating critical values. - Compute the speed and density corresponding to 1000 veh/hr. | 8M | CO-1 | BL-4 |
| UNIT-2 |  |  |  |  |  |
|  | Spot speed studies were carried out a certain stretch of a highway with mixed traffic flow and consolidated data collected are given below. Draw % cumulative frequency distribution graph and find the safe speed limit. Speed range, kmph; No. of vehicles observed; Speed range, kmph; No. of vehicles observed. 0 to 10; 16; 50 to 60; 270. 10 to 20; 22; 60 to 70; 130. 20 to 30; 76; 70 to 80; 56. 30 to 40; 98; 80 to 90; 42. 40 to 50; 240; 90 to 100; 12. | 8M | CO-1 | BL-4 |
| OR |  |  |  |  |  |
|  | The consolidated data collected from speed and delay studies by moving car method on a stretch of urban road of length 3.5km, running North- South are given below. Determine the average values of (i) Volume, (ii) Journey speed and (iii) Running speed of the traffic stream along each direction. Trip no.; Direction of trip; Journey time, min-sec; Total stopped delay, min-sec; No. of vehicles overtaking; No. of vehicles overtaken; No. of vehicles from opposite direction. 1; N-S; 6-32; 1-40; 4; 7; 268. 2; S-N; 7-14; 1-50; 5; 3; 186. 3; N-S; 6-50; 1-30; 5; 3; 280. 4; S-N; 7-40; 2-00; 2; 1; 200. 5; N-S; 6-10; 1-10; 3; 5; 250. 6; S-N; 8-00; 2-22; 2; 2; 170. 7; N-S; 6-28; 1-40; 2; 5; 290. 8; S-N; 7-30; 1-40; 3; 2; 160. | 8M | CO-2 | BL-3 |
| UNIT-3 |  |  |  |  |  |
|  | Explain different types of level of service with a neat diagram. | 8M | CO-2 | BL-2 |
| OR |  |  |  |  |  |
|  | Explain the relationship between speed, volume, and density. Also sketch their graphical relationships. | 8M | CO-2 | BL-2 |
| UNIT-4 |  |  |  |  |  |
|  | Explain different measures that can be implemented for preventing accidents. | 8M | CO-3 | BL-2 |
| OR |  |  |  |  |  |
|  | Explain the various aspects to be investigated during parking studies. Also explain anyone parking survey technique. | 8M | CO-3 | BL-2 |
| UNIT-5 |  |  |  |  |  |
|  | a) Mention any two advantages and disadvantages of traffic signals. b) List the various signal coordination methods and explain anyone in detail. | 4M 4M | CO-4 CO-4 | BL-2 BL-2 |
| OR |  |  |  |  |  |
|  | The average normal flow of traffic on crossroads 1 and 2 during design period are 540 and 350 PCU per hour; the saturation flow values on these roads are estimated as 1430 and 1100 PCU per hour respectively. Design two phase traffic signal with pedestrian crossing by Webster’s method. Assume lost time per phase as 2 sec. and inter-green time as 4sec. Also draw phasing and timing diagrams. | 8M | CO-4 | BL-4 |
| UNIT-6 |  |  |  |  |  |
|  | a) Mention the major air pollutants due to road traffic? Explain consequences of each. b) Discuss the measure to curtail environmental degradation due to traffic. | 4M 4M | CO-4 CO-4 | BL-2 BL-2 |
| OR |  |  |  |  |  |
|  | Describe the various environmental hazards due to traffic in urban areas. | 8M | CO-4 | BL-2 |

***