| Subject Code 19PC1CE17 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | R19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. IV Year I Semester Regular/Supplementary Examinations, November 2023

WASTEWATER ENGINEERING

(CE)

Time: 3 hours                                                                                 		      		Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

PART-A

6X2=12M

|  | Compare dry weather flow and wet weather flow. | 2M | CO-1 | BL-3 |
| --- | --- | --- | --- | --- |
|  | Distinguish between self cleansing and non scouring velocity in sewers. | 2M | CO-1 | BL-3 |
|  | List any 4 commonly used coagulants for sedimentation aided with coagulation process. | 2M | CO-2 | BL-1 |
|  | Draw neat sketch of soak pit and name the parts. | 2M | CO-3 | BL-3 |
|  | Explain the process of self purification of surface water bodies. | 2M | CO-3 | BL-2 |
|  | Write the difference between sanitary sewer and storm drain. | 2M | CO-4 | BL-2 |

PART-B

6X8=48M

| UNIT-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | a) Explain different methods for estimation of quantity of storm water. b) Describe the factors affecting the quantity of sewage flow. | 4M 4M | CO-1 CO-1 | BL-2 BL-2 |
| OR |  |  |  |  |
|  | Discuss the physical and chemical characteristics of domestic sewage. | 8M | CO-1 | BL-2 |
| UNIT-2 |  |  |  |  |
|  | a) Calculate the velocity of flow in a sewer of diameter 100cm laid in a slope of 1:1000. What will be the discharge when running half full? Assume, n = 0.013 b) Write any two methods for testing of sewer. | 4M 4M | CO-1 CO-1 | BL-3 BL-2 |
| OR |  |  |  |  |
|  | Enumerate the process of one pipe and two pipe system with a diagram. | 8M | CO-1 | BL-3 |
| UNIT-3 |  |  |  |  |
|  | a) With neat sketch explain working of oil and grease trap. b) Design a rectangular sedimentation tank for a population of 90000 with a rate of 140LPCD, 80% of which reaches treatment plant. Assume peak factor 1.2 and velocity of flow 0.3m/min. | 4M 4M | CO-2 CO-2 | BL-2 BL-3 |
| OR |  |  |  |  |
|  | Draw a neat sketch and explain horizontal flow grit chamber along with the design criteria. | 8M | CO-2 | BL-2 |
| UNIT-4 |  |  |  |  |
|  | a) Describe the working process of a trickling filter by using a diagram. b) List the advantages and disadvantages of septic tank. | 4M 4M | CO-2 CO-3 | BL-2 BL-1 |
| OR |  |  |  |  |
|  | Explain the significance of following parameters with reference to activated sludge process: - Hydraulic retention time (HRT) ii) F/M Ratio iii) Sludge volume index (SVI) iv) MLSS | 8M | CO-2 | BL-2 |
| UNIT-5 |  |  |  |  |
|  | a) Describe the factors favoring disposal of effluent by dilution. b) With neat sketch explain oxygen sag curve. | 4M 4M | CO-3 CO-3 | BL-2 BL-3 |
| OR |  |  |  |  |
|  | a) Write about the factors affecting sludge digestion. b) With neat sketch explain working of sludge digestion tank. | 4M 4M | CO-3 CO-3 | BL-2 BL-3 |
| UNIT-6 |  |  |  |  |
|  | a) Discuss the wastewater characteristics of a diary industry. b) With flow diagram explain wastewater treatment process for textile industry. | 4M 4M | CO-4 CO-4 | BL-2 BL-3 |
| OR |  |  |  |  |
|  | a) Explain the wastewater characteristics of a Tannery industry. b) With flow diagram explain wastewater treatment process for sugar industry. | 4M 4M | CO-4 CO-4 | BL-2 BL-3 |

***