| Subject Code 19PE1EC19 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | R19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. IV Year II Semester Regular Examinations, May 2023

DSP PROCESSORS AND ARCHITECTURES

(ECE)

Time: 3 hours                                                                                 		      	 Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

PART-A

6X2=12M

|  | What is the range of signed fractional number that can be represented in a fixed point format using 16bits? | 2M | CO-2 | BL-2 |
| --- | --- | --- | --- | --- |
|  | What is the pipeline depth and explain the effect of it on performance? | 2M | CO-1 | BL-1 |
|  | Differentiate uniform and nonuniform quantization. | 2M | CO-1 | BL-1 |
|  | What is the function of PAGEN unit in TMS 320C54XX processor? | 2M | CO-4 | BL-1 |
|  | What is the use of IOSTRB signal in parallel IO interfacing? | 2M | CO-3 | BL-1 |
|  | What is the use of interpolation and decimation filters in DSP? | 2M | CO-3 | BL-1 |

PART-B

6X8=48M

UNIT-1

| UNIT-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | - Compute the dynamic range and the percentage resolution for a block floating point format with 4-bit exponent used in a 16-bit point processor. - Discuss the benefits of VLIW architecture. | 4M 4M | CO-2 CO-1 | BL-3 BL-1 |
| OR |  |  |  |  |
|  | - Prove that the dynamic range of signal increases by 6dB for each additional bit used to represent its value. - Explain about fixed point operation with example. | 4M 4M | CO-2 CO-2 | BL-4 BL-2 |
| UNIT-2 |  |  |  |  |
|  | - Describe the unique features of programmable DSPs to handle interrupts. - Write a shot notes on relative branch support.in programmable DSPs. | 4M 4M | CO-1 CO-1 | BL-2 BL-1 |
| OR |  |  |  |  |
|  | - Discuss the effects of interrupts on the performance of pipeline in DSP processors. - Explain different pipeline programming models in brief. | 4M 4M | CO-1 CO-2 | BL-2 BL-2 |
| UNIT-3 |  |  |  |  |
|  | - Draw the block diagram of a DSP processor and explain each block in detail. - Discuss how do you compensate DAC errors in signal processing. | 4M 4M | CO-1 CO-2 | BL-2 BL-2 |
| OR |  |  |  |  |
|  | - Describe the errors introduced due to A/D converter and hence compute the signal to noise ratio. - b) Discuss the real time implementation considerations of signal processing. | 4M 4M | CO-2 CO-1 | BL-3 BL-2 |
| UNIT-4 |  |  |  |  |
|  | - Illustrate the addressing modes of 54xx processor. - Develop an assembly language program to compute the equation. | 4M 4M | CO-3 CO-3 | BL-2 BL-3 |
| OR |  |  |  |  |
|  | - Demonstrate the arithmetic instructions of 54XX processor. - Write a short notes on available on chip peripherals of 54XX processors. | 4M 4M | CO-3 CO-3 | BL-2 BL-1 |
| UNIT-5 |  |  |  |  |
|  | - Explain how MCBSP provides an interface to serial port and DSP. - Interface the TMS320C54XX to a 10 bit ADC and an 8 bit DAC. The sampled signal read from the ADC is to be written to the DAC after adjusting its size. The start of the conversion is to be initiated by the Tout signal of the timer. | 4M 4M | CO-3 CO-3 | BL-2 BL-3 |
| OR |  |  |  |  |
|  | - Represent each of the following as 16 bit number in the desired Q notation - 0.3125 as Q15 number - -0.3125 as Q15 number - 3.125 as Q7 number - -352 as Q0 number - Write a program to implement FIR filter. | 4M 4M | CO-2 CO-3 | BL-2 BL-3 |
| UNIT-6 |  |  |  |  |
|  | - List the features of TMS320C60XX processor. - Compare fixed point and floating point processors. | 4M 4M | CO-4 CO-4 | BL-1 BL-2 |
| OR |  |  |  |  |
|  | - Describe the memory architecture of TMS320C6000 processor. - Explain logical instructions of TMS320C6000 processor with sufficient examples. | 4M 4M | CO-4 CO-4 | BL-2 BL-2 |

***