| Subject Code 19PE1CE21 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | R19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. IV Year II Semester Regular Examinations, May 2023

PAVMENT ANALYSIS AND DESIGN

(CE)

Time: 3 hours                                                                                 		      	 Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

PART-A

6X2=12M

|  | List the different types of pavements | CO-1 | BL-1 |
| --- | --- | --- | --- |
|  | Give the equation for determining the deflection of pavement surface in single layer system | CO-2 | BL-1 |
|  | What are the different types of stresses in rigid pavements | CO-2 | BL-1 |
|  | What are the different methods of flexible pavement design? | CO-3 | BL-1 |
|  | What are the different types of CC pavements | CO-3 | BL-1 |
|  | What are different factors causing failure of rigid pavement | CO-4 | BL-1 |

PART-B

6X8=48M

| UNIT-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | a) Explain in detail on variables considered for the pavement design. b) Discuss briefly on the limitations of rigid pavements. | 4M 4M | CO-1 CO-1 | BL-2 BL-2 |
| OR |  |  |  |  |
|  | a) Outline in detail on the ESWL. b) Illustrate the components of rigid pavements with a neat sketch. | 4M 4M | CO-1 CO-1 | BL-2 BL-2 |
| UNIT-2 |  |  |  |  |
|  | Given a three-layered system with a = 5.9in, q=140psi, h1=7in, h2=6in, E1=400,000 psi, E2=20,000 psi and E3 = 10,000 psi. Determine all the stresses and strains at the interfaces on the axis of symmetry. | 8M | CO-2 | BL-4 |
| OR |  |  |  |  |
|  | What are the different design approaches of flexible pavement? Discuss on the advantages and disadvantages of any one method. | 8M | CO-2 | BL-2 |
| UNIT-3 |  |  |  |  |
|  | a) Compute the equivalent radius of resisting section of 20cm thick slab, given that the radius of contact area wheel is 15cm. b) Explain briefly on the critical positions of loading on CC pavements. | 4M 4M | CO-2 CO-2 | BL-3 BL-2 |
| OR |  |  |  |  |
|  | a) Write a short note on the radius of relative stiffness. b) Compute the radius of relative stiffness of 15cm thick cement concrete slab using the following data Modulus of elasticity of cement concrete =2.1×105 kg/cm2 Posisson’s ratio for concrete = 0.15 Modulus of subgrade reaction, k = 3.5 kg/cm3 | 4M 4M | CO-2 CO-2 | BL-2 BL-3 |
| UNIT-4 |  |  |  |  |
|  | Illustrate the design approach of flexible pavement as per IRC guidelines. | 8M | CO-3 | BL-3 |
| OR |  |  |  |  |
|  | Compute CSA of a new flexible pavement for a two-lane undivided carriageway using the following data Design CBR value of subgrade =6.0% Initial traffic on completion of construction =300 cvperday Average growth rate = 6.0% peryear Design life = 10 years VDF value = 2.5 | 8M | CO-3 | BL-3 |
| UNIT-5 |  |  |  |  |
|  | Explain the step wise procedure for design of CC pavement thickness as per IRC guidelines. | 8M | CO-3 | BL-3 |
| OR |  |  |  |  |
|  | Explain the objectives and functions of Dowel bars. | 8M | CO-3 | BL-2 |
| UNIT-6 |  |  |  |  |
|  | Discuss briefly on the concept of white topping. | 8M | CO-4 | BL-2 |
| OR |  |  |  |  |
|  | Outline on the strengthening existing CC pavement by overlays. | 8M | CO-4 | BL-3 |

***