| Subject Code 19PE1CE29 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | R19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. IV Year II Semester Regular Examinations, May 2023

PRE-ENGINEERED BUILDINGS

(CE)

Time: 3 hours                                                                                 		      	 Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

Assume any other missing data suitably.

Use of IS: 800 and steel tables are permitted.

PART-A

6X2=12M

|  | Specify the Indian Standard codes used for the design of PEB. |  | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | What are the components of secondary system in PEB? |  | CO-1 | BL-1 |
|  | What is the importance of serviceability limits as per the IS code? |  | CO-2 | BL-2 |
|  | Write the relation between flange thickness to web thickness. |  | CO-2 | BL-2 |
|  | What is the effect of shear buckling in columns? |  | CO-3 | BL-2 |
|  | Write the equation to calculate the bearing pressure in concrete. |  | CO-4 | BL-1 |

PART-B

6X8=48M

| UNIT-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | a) Discuss in detail about the evolution of pre-engineered buildings. b) What are the materials used for manufacturing of PEB and explain. | 4M 4M | CO-1 CO-1 | BL-2 BL-1 |
| OR |  |  |  |  |
|  | a) Explain in detail about various applications of PEB over conventional steel building. b) What are the steps to be followed in the construction of PEB? | 4M 4M | CO-1 CO-1 | BL-2 BL-2 |
| UNIT-2 |  |  |  |  |
|  | a) Explain the major components of PEB. b) Write a short notes on bracing systems. | 4M 4M | CO-2 CO-2 | BL-2 BL-1 |
| OR |  |  |  |  |
|  | a) Describe the various accessories used in PEB. b) Explain in detail about the roofing and cladding systems. | 4M 4M | CO-2 CO-2 | BL-2 BL-2 |
| UNIT-3 |  |  |  |  |
|  | Define the design load and explain various types of loads acting on the PEB. | 8M | CO-3 | BL-1 |
| OR |  |  |  |  |
|  | Design a pre-engineered building frame for wind load for a factory building of span 20 m and a pitch of 1/15. The height of the truss at eves is 10 m. The spacing of the trusses is 5.5 m. The factory building, which is 36 m long, is situated at Delhi. Take building openings as 10%. Assume K1 = 1.0, K2 = 0.91, K3 = 1.0, K4 = 1.15. | 8M | CO-3 | BL-3 |
| UNIT-4 |  |  |  |  |
|  | Discuss in detail about the design parameters of PEB frames as per IS codes. | 8M | CO-3 | BL-2 |
| OR |  |  |  |  |
|  | Design an I-section of span 20 m used in PEB where flange thickness is limited to 8 mm. | 8M | CO-3 | BL-3 |
| UNIT-5 |  |  |  |  |
|  | Evaluate the axial compression capacity of column with I-section having flange size 250 mm x 12 mm and web size 1200 x 8mm. Take E = 2 X 105 N/mm2, Grade of steel as 345 N/mm2. | 8M | CO-4 | BL-2 |
| OR |  |  |  |  |
|  | Calculate the shear capacity of the member (300 mm x 12 mm) for Flange, 450 mm x 8 mm for web, when the web does not fail due to shear. Take grade of steel as 345 N/mm2. | 8M | CO-4 | BL-3 |
| UNIT-6 |  |  |  |  |
|  | Discuss the detailed design procedure of rigid frame moment connection. | 8M | CO-4 | BL-2 |
| OR |  |  |  |  |
|  | Design a pinned base plate to fix a column of size 400 mm x 6 mm (web) + 300 mm x 15 mm (flange). Assume the grade of bolt as 4.6. Consider the load combinations at the base as 200 kN downward force with 30 kN shear force and 175 kN uplift force with 60 kN shear force. | 8M | CO-4 | BL-3 |

***