| Subject Code 19PE1CE27 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | R19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. IV Year II Semester Regular Examinations, May 2023

PRE-STRESSED CONCRETE

(CE)

Time: 3 hours                                                                                 		      	Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

PART-A

6X2=12M

|  | List the applications of prestressed concrete. |  | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | List the devices used for tensioning of steel in prestressed concrete? |  | CO-2 | BL-1 |
|  | What do you mean by loss of prestress? |  | CO-3 | BL-2 |
|  | List the basic assumptions in analysis of prestressed concrete members. |  | CO-3 | BL-1 |
|  | What are the factors influencing the deflection of prestressed concrete members. |  | CO-4 | BL-2 |
|  | List any two advantages of composite construction. |  | CO-4 | BL-1 |

PART-B

6X8=48M

| UNIT-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | a) Why are high strength concrete and high grade steel required for prestressed concrete? b) What are the advantages of prestressed concrete construction? | 4M 4M | CO-1 CO-1 | BL-2 BL-2 |
| OR |  |  |  |  |
|  | Explain the different types of prestressing. | 8M | CO-1 | BL-2 |
| UNIT-2 |  |  |  |  |
|  | Discuss about any two types of post tensioning anchorage systems with neat sketches. | 8M | CO-2 | BL-2 |
| OR |  |  |  |  |
|  | List the advantages and disadvantages of pre tensioning and post tensioning systems. | 8M | CO-2 | BL-2 |
| UNIT-3 |  |  |  |  |
|  | Discuss in detail about various types of losses in prestress. | 8M | CO-3 | BL-2 |
| OR |  |  |  |  |
|  | A prestressed concrete beam 200 mm wide and 300 mm deep is prestressed with wires (Area = 320 mm2) located at 50 mm from the bottom carrying an initial stress of 1000 N/mm2. The span of the beam is 10 m. Calculate the percentage loss of prestress in wires when the beam is post-tensioned. Assume Es = 210 kN/mm2, Ec = 35 N/mm2. Relaxation of steel stress = 5% initial stress, shrinkage of concrete = 200 × 10-6, creep coefficient = 1.6, slip at anchorage = 1 mm, friction coefficient = 0.0015 per metre. | 8M | CO-3 | BL-3 |
| UNIT-4 |  |  |  |  |
|  | A post-tensioned prestressed concrete T-beam having a flange width of 1200 mm and flange thickness of 200 mm thickness of web being 300 mm is prestressed by 2000 mm2 of high tensile steel located at an effective depth of 1600 mm. If fck= 40 N/mm2 and fp=1600 N/mm2, estimate the ultimate flexural strength of the unbounded T-section assuming span/depth ratio as 20 and fpe=1000 N/mm2. | 8M | CO-3 | BL-3 |
| OR |  |  |  |  |
|  | A rectangular concrete beam of cross section 300 mm deep and 200 mm wide is prestressed by means of 15 wires of 5 mm diameter located at 65 mm from the bottom of the beam and 3 wires of 5 mm diameter at 25 mm from the top. Assume the prestress in stell as 840 MPa. Calculate the stresses in extreme fibres at the mid-span section when the beam is supporting a uniformly distributed load of 6 kN/m in addition to its self-weight over a span of 6 m. | 8M | CO-3 | BL-4 |
| UNIT-5 |  |  |  |  |
|  | A concrete beam with a rectangular section 300 mm x 500 mm is prestressed by two post tensioned cables of area 600 mm2 each, initially stressed to 1600 N/mm2. The cables are located at a constant eccentricity of 100 mm throughout the length of the beam having a span of 10 m. The modulus of elasticity of steel and concrete are 210 and 38 kN/mm2 respectively. (i) Neglecting all losses, find the deflection at the centre of span when it is supporting its own weight. (ii) Allowing for 20% loss in prestress, find the deflection at the centre of span when it carries an imposed load of 18 kN/m. | 8M | CO-3 | BL-4 |
| OR |  |  |  |  |
|  | a) What is the necessity of control on deflections in prestressed concrete members? b) What are the factors influencing deflections? | 4M 4M | CO-4 CO-4 | BL-2 BL-2 |
| UNIT-6 |  |  |  |  |
|  | A composite beam of rectangular section is made up of a prestressed inverted T-beam having a slab thickness and width of 150 mm and 1000 mm respectively. The rib size is 150 mm by 850 mm. The cast in situ concrete has a thickness and width of 1000 mm with a modulus of elasticity of 30 kN/mm2. If the differential shrinkage is 100 X 10-6 units, estimate the shrinkage stresses developed in the precast and cast in situ units. Differential shrinkage, ecs = 100 X 10-6. | 8M | CO-4 | BL-4 |
| OR |  |  |  |  |
|  | a) What is differential shrinkage in composite construction? b) A precast post tensioned beam of rectangular section has a breadth of 100 mm and depth of 200 mm. The beam with an effective span of 5 m is prestressed by tendons with their centroids coinciding with the bottom kern. The initial force in tendons is 150 kN. The loss of prestress maybe assumed as 15%. The beam is incorporated in a composite T-Beam by casting a top flange of breadth 400 mm and thickness 40 mm. If the composite beam supports a live load of 8 kN/m2, calculate the resultant stresses developed assuming the prestressed beam as unpropped. Assume same modulus of elasticity for concrete in precast beam and insitu cast slab. | 2M 6M | CO-4 CO-4 | BL-2 BL-3 |

***