| Subject Code 19PE1EC16 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | R19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. IV Year II Semester Regular Examinations, May 2023

RADAR SYSTEMS

(ECE)

Time: 3 hours                                                                                 		      	 Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

PART-A

6X2=12M

|  | What do you mean by maximum unambiguous range? |  | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | Calculate the minimum pulse interval and pulse repetition frequency required for a radar to detect unambiguous targets up to a range of 125 miles. |  | CO-1 | BL-4 |
|  | Define blind speeds? |  | CO-2 | BL-4 |
|  | What are the advantages of Mono pulse tracking radar |  | CO-3 | BL-3 |
|  | List the applications of phased array Radar. |  | CO-3 | BL-2 |
|  | Define Noise figure |  | CO-4 | BL-2 |

PART-B

6X8=48M

| UNIT-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Derive the radar range equation. Discuss the effect of pulse width and pulse repetition frequency on the performance of radar. | 8M | CO-1 | BL-4 |
| OR |  |  |  |  |
|  | Draw the block diagram of basic radar and explain its working | 8M | CO-1 | BL-2 |
| UNIT-2 |  |  |  |  |
|  | - What is Doppler effect? A C-band (fT = 5000MHz) Doppler Radar is to detect all target with radial velocities greater than 5 miles per hour and less than 60 miles per hour. What are the minimum and maximum Doppller frequencies which Radar must detect? - Draw the block diagram of an FMCW altimeter and explain its operation. | 3M 5M | CO-2 CO-2 | BL-4 BL-2 |
| OR |  |  |  |  |
|  | Show that a CW Radar can determine target range but of no practical use. How it can measure the range and velocity using FM CW Radar? | 8 M | CO-2 | BL-4 |
| UNIT-3 |  |  |  |  |
|  | Draw the block diagram of MTI radar with power amplifier transmitter and explain its operation. | 8 M | CO-4 | BL-2 |
| OR |  |  |  |  |
|  | Describe the operation of an MTI Radar with the help of a neat block diagram. | 8 M | CO-3 | BL-3 |
| UNIT-4 |  |  |  |  |
|  | What is the significance of the term “monopulse”? Describe the operation of an amplitude comparison monopulse radar with a neat block diagram. | 8 M | CO-3 | BL-4 |
| OR |  |  |  |  |
|  | Explain sequential lobing, conical scan and monopulse techniques. Discuss their relative merits. | 8 M | CO-2 | BL-2 |
| UNIT-5 |  |  |  |  |
|  | Discuss the relationship between matched filter characteristics and correlation function? | 8M | CO-4 | BL-4 |
| OR |  |  |  |  |
|  | Write about the Matched Filter with Non-white Noise and explain? | 8M | CO-4 | BL-2 |
| UNIT-6 |  |  |  |  |
|  | Explain the different types of radar displays. | 8M | CO-2 | BL-3 |
| OR |  |  |  |  |
|  | Explain the different types of radar duplexers | 8M | CO-2 | BL-3 |