| Subject Code 22BS1CH101 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. I Year I Semester Regular / Supplementary Examinations, January 2025

ENGINEERING CHEMISTRY

(ME)

Time: 3 hours                                                          			      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | Among the chloramine and Bleaching powder, which is the powerful disinfectant? Why? | 2M | CO-1 | BL-3 |
| --- | --- | --- | --- | --- |
|  | Define the conducting polymer with example? | 2M | CO-2 | BL-3 |
|  | What are the batteries? How are they classified? | 2M | CO-3 | BL-2 |
|  | “Rusting of iron is more in saline water” explain. | 2M | CO-4 | BL-3 |
|  | Define the lubricants and list out the four important characteristics of good lubricant. | 2M | CO-5 | BL-2 |

PART-B				5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | - Explain in detail (principle and procedure) how hardness of a water sample is estimated by EDTA method. | 6M | CO-1 | BL-2 |
|  | - Calculate the permanent hardness and temporary hardness of a water sample containing the following salts, MgCO3 = 84 mg/L, Ca(HCO3)2= 40.5mg/L, CaCl2=111mg/L, MgSO4=60mg/L. | 4M | CO-1 | BL-3 |
| OR |  |  |  |  |
| 2. | - What are the factors that lead to caustic embrittlement in boilers? How can this have prevented? | 4M | CO-1 | BL-2 |
|  | - What is the desalination? Explain the desalination of brackish water by Reverse osmosis method. | 6M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |
| 3. | - Distinguish between chain growth and step growth polymerization. | 4M | CO-2 | BL-2 |
|  | - Discuss about the preparation and properties of Teflon. | 6M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 4. | - What are shape memory polymers? Explain the features and applications of shape memory polymers? | 6M | CO-2 | BL-2 |
|  | - Write the four differences between thermo plastics and thermo setting plastics with examples. | 4M | CO-2 | BL-2 |
| UNIT-III |  |  |  |  |
| 5. | Explain in detail how is coal analyzed by ultimate analysis method and give its significance. | 10M | CO-3 | BL-2 |
| OR |  |  |  |  |
| 6. | - Why lithium-ion batteries are preferred in electric vehicles? Explain. | 4M | CO-3 | BL-3 |
|  | - Explain the construction and working of methanol and oxygen fuel cell. | 6M | CO-3 | BL-2 |
| UNIT-IV |  |  |  |  |
| 7. | - What is galvanic corrosion and how it can be prevented. | 4M | CO-4 | BL-2 |
|  | - Explain the electro chemical theory of corrosion of metals with special reference to rusting of iron. | 6M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 8. | - Discuss the important factors that influence the corrosion phenomenon. | 6M | CO-4 | BL-2 |
|  | - Write note on sacrificial anode and impressed current cathodic protection. | 4M | CO-4 | BL-2 |
| UNIT-V |  |  |  |  |
| 9. | - Write the brief note on Glass, Fibre and aramid reinforced composites. | 6M | CO-5 | BL-2 |
|  | - What is the importance of Flash and Fire point. | 4M | CO-5 | BL-2 |
| OR |  |  |  |  |
| 10. | - What are self-healing materials? Discuss the principle and applications of self-healing materials. | 6M | CO-5 | BL-2 |
|  | - What are Biodegradable lubricants? How are they superior to conventional lubricants. | 4M | CO-5 | BL-2 |

***