| Subject Code 22ES1EE101 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. I Year II Semester Regular/Supplementary Examinations, June 2025

BASIC ELECTRICAL AND ELECTRONICS ENGINEERING

(Common to AIML, IOT and AIDS)

Time: 3 hours                                                                                 			      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | What is the total resistance of four 10 Ω resistors connected in parallel? | 2M | CO-1 | BL-3 |
| --- | --- | --- | --- | --- |
|  | What is the phase difference between voltage and current in a purely inductive AC circuit? | 2M | CO-2 | BL-1 |
|  | Define voltage regulation of a transformer. | 2M | CO-3 | BL-1 |
|  | Define slip in an induction motor and write the expression for it. | 2M | CO-3 | BL-2 |
|  | Mention the applications of diode. | 2M | CO-5 | BL-1 |

PART-B					5X10=50M

| UNIT-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | Find the current flowing through 20 Ω resistor using the superposition theorem | 10M | CO-1 | BL-3 |
| OR |  |  |  |  |
| 2. | - State and explain Kirchhoff’s laws. - List various types of dependent sources and draw their symbols. | 5M 5M | CO-1 CO-1 | BL-2 BL-2 |
| UNIT-II |  |  |  |  |
| 3. | - Analyze a series R-C circuit fed with a single-phase AC supply and draw the phasor diagram for the circuit. - A series R-L circuit having a resistance of 4Ω and 3 ohms inductive reactance is fed by 100V, 50Hz, single phase supply. Find the current flowing through the circuit and power factor offered by the circuit. | 5M 5M | CO-2 CO-2 | BL-3 BL-3 |
| OR |  |  |  |  |
| 4. | Derive the mathematical expressions for average value and RMS value of an alternating sinusoidal voltage given by v(t)=Vmsin(t). | 10M | CO-2 | BL-3 |
| UNIT-III |  |  |  |  |
| 5. | Draw a neat sketch describing to basic construction of a DC machine. Explain the function of each part of the machine. | 10M | CO-3 | BL-2 |
| OR |  |  |  |  |
| 6. | - Explain the working principle of a transformer with neat diagrams. - Differentiate between an ideal transformer and a practical transformer. | 5M 5M | CO-3 CO-3 | BL-2 BL-2 |
| UNIT-IV |  |  |  |  |
| 7. | - Classify and explain about various types of 3-phase induction motors based on the rotor construction with neat sketches. | 5M | CO-3 | BL-2 |
|  | - Describe the working principle of Synchronous generator with neat sketches. | 5M | CO-3 | BL-2 |
| OR |  |  |  |  |
| 8. | What is the need of earthing? Illustrate the pipe earthing and plate earthing schemes with neat diagrams. | 10M | CO-4 | BL-2 |
| UNIT-V |  |  |  |  |
| 9. | - Draw and explain the V-I characteristics (forward and reverse bias) of a P-N junction diode. | 5M | CO-5 | BL-2 |
|  | - Describe the construction of bipolar junction transistor (BJT) with neat diagram. | 5M | CO-5 | BL-2 |
| OR |  |  |  |  |
| 10. | Describe the operation of full wave rectifier (center tap and bridge rectifiers) with relevant waveforms. | 10M | CO-5 | BL-2 |

***