| Subject Code 22PC1EC101 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. I Year II Semester Regular/Supplementary Examinations, June 2025

ELECTRONIC DEVICES AND CIRCUITS

(ECE)

Time: 3 hours                                                                                 			      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | What is diode? give its applications | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | What is filter? What is the need of filters in rectifier circuit? | 2M | CO-1 | BL-1 |
|  | What do you mean by Early effect? | 2M | CO-3 | BL-2 |
|  | Classify Field effect Transistors. | 2M | CO-3 | BL-1 |
|  | What are the advantages of self-bias circuit? | 2M | CO-4 | BL-2 |

PART-B					5X10=50M

| UNIT-I |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1. | - Explain the operation of a PN junction diode in forward biased and reverse biased condition | 5M | CO-1 | BL-2 |
|  | - Explain the terms ‘Avalanche breakdown’ and ‘Zener Breakdown’. | 5M | CO-2 | BL-1 |
| OR |  |  |  |  |  |
| 2. | - Draw and explain the following diode equivalent models. a) Piecewise-Linear model b) simplified model c) ideal model | 5M | CO-1 | BL-2 |
|  | - Describe how diffusion and transition capacitances differ and derive the expression for transition capacitance. | 5M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |  |
| 3. | - Draw and explain the working of half wave rectifier. | 5M | CO-1 | BL-1 |
|  | - Derive the ripple factor and efficiency of Half Wave Rectifier. | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |  |
| 4. | A 230V, 50HZ voltage is applied to primary of a 5:1 step down center tap transformer used in Full Wave Rectifier with load resistance of 5KΩ. Calculate the following i) Maximum DC Voltage ii) R.M.S iii) Ripple Factor iv) PIV v) Efficiency | 10M | CO-1 | BL-3 |
| UNIT-III |  |  |  |  |  |
| 5. | - Explain the operation of a Transistor in CE configuration using its i/p and o/p characteristics. | 5M | CO-3 | BL-2 |
|  | - The common base d.c current gain of transistor is 0.967. If the emitter current is 10mA, what is the value of the base current? | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |  |
| 6. | - Derive general equation for collector current, IC in CB and CE configuration? | 5M | CO-3 | BL-2 |
|  | - A transistor operating in CB configuration has IC = 2.98mA, IE = 3mA and IC0 = 0.01mA. What current will flow in the collector circuit of this transistor when connected in CE configuration with base current of 30µA. | 5M | CO-3 | BL-3 |
| UNIT-IV |  |  |  |  |  |
| 7. | - Derive the following parameters of the JFET and relate them. - gm - rd - µ | 5M | CO-3 | BL-2 |
|  | - Explain the depletion mode of operation in MOSFET with neat diagrams? | 5M | CO-3 | BL-2 |
| OR |  |  |  |  |  |
| 8. | - Draw and explain the operation of P-channel JFET | 5M | CO-3 | BL-2 |
|  | - Explain the construction & operation of an N-channel MOSFET in enhancement mode with the help of static drain characteristics and transfer characteristics? | 5M | CO-3 | BL-2 |
| UNIT-V |  |  |  |  |  |
| 9. | - Define Stability factors S, S’ and S” and derive the expression for S. | 5M | CO-4 | BL-2 |
|  | - Derive the stability factor S for fixed bias circuit. | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |  |
| 10. | - Draw and explain the self-bias circuit with the proper equations. | 5M | CO-4 | BL-2 |
|  | - For the emitter bias circuit as shown in below Fig., determine: (a) IB (b) IC (c) VCE (d) Stability factor. | 5M | CO-4 | BL-3 |

****