| Subject Code 22ES1ME101 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. I Year II Semester Regular / Supplementary Examinations, June 2025

ENGINEERING MECHANICS

(ME)

Time: 3 hours                                                                                 		                Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | Give a practical example of concurrent force system in daily life with a neat figure. | 2M | CO-1 | BL-3 |
| --- | --- | --- | --- | --- |
|  | What is angle of repose? | 2M | CO-2 | BL-1 |
|  | Different between curvilinear motion and rectilinear motion. | 2M | CO-2 | BL-4 |
|  | State work energy principle. | 2M | CO-3 | BL-1 |
|  | State impulse and momentum equation. | 2M | CO-4 | BL-1 |

PART-B					5X10=50M

| UNIT-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | Find the resultant and direction of the force system, if the forces of 20N, 30N, 40N, 50N and 60N are acting from one of the corner and acts towards the other five corners of a regular Hexagon. | 10M | CO-1 | BL-4 |
| OR |  |  |  |  |
| 2 | Two identical rollers, each weighing 500 N, are supported by an inclined plane making an angle of 30o to the horizontal and a vertical wall. Assuming smooth surfaces, find all the possible reactions drawing a neat sketch. | 10M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |
| 3 | A load of 400 N is lying on an inclined plane, whose inclination with the horizontal is 45°. If the coefficient of friction between the load and the plane is 0.25, find the least horizontal force, which will keep the load in equilibrium. | 10M | CO-1 | BL-4 |
| OR |  |  |  |  |
| 4 | Find the centroid of the plain lamina shown in Fig. | 10M | CO-2 | BL-4 |
| UNIT-III |  |  |  |  |
| 5 | A girder of I shape cross-section has equal flanges each 120mm x 20 mm connected by a web 200mmx20mm. Determine the moment of inertia about its centroidal axis which is parallel to the flanges. | 10M | CO-2 | BL-4 |
| OR |  |  |  |  |
| 6 | Determine the moment of inertia about the centroidal axes of a solid cone of radius R, height h and mass m. | 10M | CO-2 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7 | A ball is thrown so that it just clears a 8.5m wall 32m away. If it left the 1.5m above the ground and at an angle of 600 to the horizontal. What is the initial velocity of the ball. | 10M | CO-3 | BL-4 |
| OR |  |  |  |  |
| 8 | A block of mass 30 kg is resting on a horizontal table 1.5 m from its edge. The block A is attached to the string whose other end is carrying a body B of mass 3 kg. If the coefficient of friction between the block A and the table is 0.12, find the acceleration of each block and tension in the string. | 10M | CO-3 | BL-4 |
| UNIT-V |  |  |  |  |
| 9 | A rectangular block 10 N is resting on a rough horizontal plane. This block is attached to another weight of 8 N attached with a cord and hanging over a smooth pulley vertically. If the coefficient of friction of the horizontal surface is 0.2, find (a) Acceleration of the system (b) Tension in the cord . The weight of 10 N is 1.5 m away from the pulley. | 10M | CO-4 | BL-4 |
| OR |  |  |  |  |
| 10 | A body of weight 100 N rests on a rough included plane of 10o inclination to the horizontal. It is pilled up by a cord which passes over the pulley mounted at the end of the inclined plane and carries a weight of 800N and hangs vertically. Taking the coefficient of friction µ between the plane and 100 N weight is 0.2, determine (a) The tension in the string (b) The acceleration of the system and (c) The distance moved by the body is 4 seconds starting from rest. | 10M | CO-4 | BL-4 |