| Subject Code 22ES1EC101 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. I Year II Semester Regular/Supplementary Examinations, June 2025

PRINCIPLES OF ELECTRONICS ENGINEERING

(CSBS)

Time: 3 hours                                                                                 			      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | What is the difference between drift current and diffusion current in a semiconductor? | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | What is meant by stability factor in transistor biasing, and why is it important? | 2M | CO-2 | BL-2 |
|  | Write the differences between depletion-type and enhancement-type MOSFETs. | 2M | CO-3 | BL-2 |
|  | List out different types of the feedback amplifiers. | 2M | CO-4 | BL-1 |
|  | Discuss the function of a demultiplexer? | 2M | CO-5 | BL-1 |

PART-B					5X10=50M

| UNIT-I |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1. | - Explain the formation of a P-N junction diode. How does the depletion region develop, and what role does the built-in potential play in the operation of the junction? | 5M | CO-1 | BL-2 |
|  | - Write the differences between avalanche and Zener breakdowns. Also explain Zener diode operation with diagram and V-I characteristics. | 5M | CO-1 | BL-3 |
| OR |  |  |  |  |  |
| 2. | What is ripple factor? Derive the ripple factor for a Half-Wave and Full-Wave Rectifier and also determine the efficiency of both rectifiers. | 10M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |  |
| 3. | - Explain the input and output characteristics of a transistor in Common Emitter (CE) configuration. | 5M | CO-2 | BL-2 |
|  | - Compare transistor CB, CE and CC configurations and determine the relation between , , parameters. | 5M | CO-2 | BL-4 |
| OR |  |  |  |  |  |
| 4. | What is transistor biasing? Explain why biasing is necessary. Discuss the effects of temperature on transistor parameters and derive the expression for the stability factor in a Fixed Bias circuit. | 10M | CO-2 | BL-3 |
| UNIT-III |  |  |  |  |  |
| 5. | Explain the construction and working of an N-channel JFET. Draw and describe its drain and transfer characteristics. | 10M | CO-3 | BL-3 |
| OR |  |  |  |  |  |
| 6. | Discuss the structure and operation of a CMOS inverter. Explain why CMOS circuits have low power consumption and write its advantages over Bipolar. | 10M | CO-3 | BL-5 |
| UNIT-IV |  |  |  |  |  |
| 7. | - With the help of block diagram explain the concept of feedback amplifier. | 5M | CO-4 | BL-2 |
|  | - What are the advantages of negative feedback in amplifiers? | 5M | CO-4 | BL-3 |
| OR |  |  |  |  |  |
| 8. | Draw the circuit diagram of an inverting amplifier using an op-amp. Derive the expression for its voltage gain and explain its working. | 10M | CO-4 | BL-3 |
| UNIT-V |  |  |  |  |  |
| 9. | - What is a multiplexer? Draw the logic diagram of a 4-to-1 multiplexer and explain its operation using truth table and selection lines. | 5M | CO-5 | BL-4 |
|  | - Describe the working of a D flip-flop with truth table and timing diagram. | 5M | CO-5 | BL-2 |
| OR |  |  |  |  |  |
| 10. | Explain the working of a 4-bit asynchronous counter using JK flip-flops. Show the timing diagram and count sequence. | 10M | CO-5 | BL-3 |

****