| Subject Code 22PC1CE102 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. I Year II Semester Regular/Supplementary Examinations, June 2025

SURVEYING

(CE)

Time: 3 hours                                                                                 			      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | What are the different types of bearing systems in compass surveying? Sketch. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | List the temporary adjustments of leveling instrument. | 2M | CO-2 | BL-2 |
|  | Define face right and face left observations. | 2M | CO-3 | BL-1 |
|  | What is the relationship between radius and degree of curve? | 2M | CO-4 | BL-1 |
|  | List out any four application of Total Station. | 2M | CO-5 | BL-1 |

PART-B					5X10=50M

| UNIT-I |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1. | - Classify and explain in detail the method of surveying based on the instruments used and purpose of surveying. | 5M | CO-1 | BL-2 |
|  | - Explain in detail about the sources of errors in surveying. | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |  |
| 2. | The following bearings were observed in running a closed traverse. Line; F.B; B.B. AB; 1500 0’; 329045’. BC; 77030’; 25600’. CD; 41030’; 222045’. DE; 314015’; 134045’. EA; 220015’; 40015’. Determine the included angles, error and apply corrections if any. | 10M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |  |
| 3. | - Define contour. What do you understand by contour interval, list the characteristics of contour? | 5M | CO-2 | BL-2 |
|  | - A series of offsets were taken from a chain line to a curved boundary line at intervals of 15 meters in the following order : 0.2, 0.65, 3.80, 3.75, 4.65, 3.60, 4.95, 5.85m, and 2.35. Compute the area between the chain line, the curved boundary and the end offsets by i) Trapezoidal rule ii) Simpsons rule | 5M | CO-2 | BL-3 |
| OR |  |  |  |  |  |
| 4. | The following staff readings were observed successively with a leveling instrument. The instrument was shifted after fifth and eleventh readings.0.585; 1.010; 1.735; 3.925; 3.775; 0.350; 1.300; 1.735; 2.575; 3.375; 3.895; 1.735; 0.635; 1. 605m.Draw up a page of level book and determine the various points if the R.L of the point on which the first reading was taken is 136.440. Use height of instrument & rise and fall method. | 10M | CO-2 | BL-3 |
| UNIT-III |  |  |  |  |  |
| 5. | - How to find horizontal angle using Reiteration Method? | 5M | CO-3 | BL-2 |
|  | - Determine the level of top of an object above the ground when its base is inaccessible using trigonometrical levelling. | 5M | CO-3 | BL-2 |
| OR |  |  |  |  |  |
| 6. | How will you set out a horizontal angle by method of repetition? Also show the tabular form to record the readings. | 10M | CO-3 | BL-2 |
| UNIT-IV |  |  |  |  |  |
| 7. | - A tacheometer was set up at station A and the following readings were obtained on a vertically held staff. Station; Staff Station; Vertical Angle; Hair Reading; Remarks. A; B.M B; (-) 2018’ (+) 80 36’; 3.225, 3.550, 3.875, 1.650, 2.515, 3.380; R.L of BM = 437.655m. Calculate the horizontal distance from A to B and the R.L of B, if the constants of the instrument were 100 and 0. | 5M | CO-3 | BL-4 |
|  | - Two tangents intersect at chainage 1250 m. The deflection angle is 50o. Calculate the following data necessary for setting out a curve of radius 250 m (i) Tangent Length (ii) Length of Circular curve (iii) Length of the Long Chord (iv) Chainages of the tangent points. | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |  |
| 8. | Two tangents intersect at the chainage 1190m, the deflection angle being 360.Calculate all the data necessary for setting out a curve with a radius of 300m by the Rankines deflection angle method. The peg intervals may be taken as 20 m. prepare a setting out table of data for field checking. | 10M | CO-3 | BL-4 |
| UNIT-V |  |  |  |  |  |
| 9. | - A line long lying at an elevation of 500m measures as 10.5cm on vertical photograph. The focal length of the camera used is 20cm. determine the scale of the photograph for an area having elevation of 1200m. | 5M | CO-4 | BL-2 |
|  | - What are the advantages and dis advantages of total station along with a few applications in surveying? | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |  |
| 10. | a) Write a brief note on Global positioning system | 5M | CO-4 | BL-2 |
|  | b) Discuss the concept of Electronic distance measurement. | 5M | CO-4 | BL-2 |

****