| Subject Code 22PC1EC209 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year I Semester Regular/Supplementary Examinations, December/January 2025

BASIC ELECTRONIC DEVICES AND CIRCUITS

(EEE)

Time: 3 hours                                                                                 		       Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A				5X2=10M

|  | Draw the V-I characteristics of UJT. | 2M | CO1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | Obtain the relation among α, β and γ. | 2M | CO2 | BL-2 |
|  | Define the pinch-off voltage in a JFET. | 2M | CO2 | BL-1 |
|  | Draw the small signal model of a BJT. | 2M | CO5 | BL-2 |
|  | Classify the Oscillators. | 2M | CO4 | BL-2 |

PART-B				5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | - Derive transition and diffusion capacitances for PN junction diode | 10M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 2. | Draw the circuit diagram of half wave rectifier. Explain its operation and obtain its ripple factor and rectification efficiency. | 10M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |
| 3. | - Discuss the phenomenon “Thermal run away”. Derive the condition for thermal stability. | 7M | CO-3 | BL-2 |
|  | - A transistor has I B = 100 μ A and I C = 2 m A. Find α , β of the transistor and emitter current I E. | 3M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 4. | Derive the expression for the stability factor S for self-bias circuit and discuss its significance. | 10M | CO-3 | BL-2 |
| UNIT-III |  |  |  |  |
| 5. | a) An n- channel JFET connected in common source configuration is desired to bias at ID =0.8 mA using V DD =24V is having Vgsoff = -2V and IDSS = 1.65 mA. Find VGS and gm | 6M | CO-2 | BL-3 |
|  | b) Classify FETs and also state its applications. | 4M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 6. | Explain the construction, working and V-I characteristics of n-channel enhancement-type MOSFET. | 10M | CO-2 | BL-2 |
| UNIT-IV |  |  |  |  |
| 7. | - Discuss the analysis of CS JFET Amplifier. | 5M | CO-5 | BL-2 |
|  | - Describe the small signal model for JFET. | 5M | CO-5 | BL-2 |
| OR |  |  |  |  |
| 8. | Derive the expressions for current gain, input impedance, voltage gain and output admittance of CE amplifier using exact h-parameter model. | 10M | CO-5 | BL-2 |
| UNIT-V |  |  |  |  |
| 9. | - Classify various Feedback amplifiers with Block diagrams. | 5M | CO-4 | BL-2 |
|  | - Discuss the effect of Negative Feedback on Amplifier characteristics | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 10. | A voltage-series negative feedback amplifier has a voltage gain without feedback of A = 500, input resistance Ri = 3 KΩ, output resistance Ro=20 KΩ and feedback ratio β = 0.01. Calculate the voltage gain Af, input resistance Rif and output resistance Rof of the amplifier with feedback. | 10M | CO-4 | BL-3 |

***