| Subject Code 22PC1CE202 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year I Semester Regular/Supplementary Examinations, December/January 2025

CONCRETE TECHNOLOGY

(CE)

Time: 3Hrs										      Max.Marks:60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

Use of IS: 10262: 2019 is allowed.

PART-A					5X2=10M

|  | Explain the process of hydration of cement. | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | State any two mechanical properties of aggregates. | 2M | CO-2 | BL-2 |
|  | State any two types of chemical admixtures. | 2M | CO-3 | BL-2 |
|  | Describe the maturity concept in concrete. | 2M | CO-4 | BL-2 |
|  | Write any three types of cement. | 2M | CO-4 | BL-2 |

PART-B				5X10=50M

| UNIT-I |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1. | - Describe Bogue’s compounds and explain their significance. | 5M | CO-1 | BL-2 |
|  | - Describe the chemical composition of cement and its role in determining the material's performance. | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |  |
| 2. | Enumerate the steps involved in the manufacture of Portland cement by any one method with a flow diagram. | 10M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |  |
| 3. | - Explain the phenomenon of bulking of sand and its impact on concrete behaviour. | 5M | CO-2 | BL-2 |
|  | - State the significance of fineness modulus and maximum aggregate size in concrete mix design. | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |  |
| 4. | Explain the classification of aggregates based on size, shape, and texture, and discuss their impact on concrete properties. | 10M | CO-2 | BL-2 |
| UNIT-III |  |  |  |  |  |
| 5. | - Describe the term "workability" of fresh concrete. How does water-cement ratio influence workability? | 5M | CO-3 | BL-2 |
|  | - Explain the effect of time and temperature on workability. | 5M | CO-3 | BL-2 |
| OR |  |  |  |  |  |
| 6. | Discuss the types of admixtures in modifying the properties of concrete. | 10M | CO-3 | BL-2 |
| UNIT-IV |  |  |  |  |  |
| 7. | - Describe the factors affecting the strength of concrete. | 5M | CO-4 | BL-2 |
|  | - Explain the term Gel/space ratio and its relationship with strength of concrete. | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |  |
| 8. | a) Describe phenomenon of shrinkage in concrete along with different types of shrinkage. b) Discuss factors affecting the modulus of elasticity. | 5M 5M | CO-4 CO-4 | BL-2 BL-2 |
| UNIT-V |  |  |  |  |  |
| 9. | - Discuss about self-compacting concrete and its typical applications. | 5M | CO-4 | BL-2 |
|  | - Explain about fibre reinforced concrete and its typical applications. | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |  |
| 10. | Design a concrete mix for characteristic strength of 35MPa at 28 days with a standard deviation of 5MPa.The specific gravity of FA and CA are 2.60 and 2.70 respectively. A slump of 50mm is necessary. The specific gravity of cement is 3.15. Assuming the necessary data design the mix as per IS code method. Type of exposure is mild, water absorption for coarse and fine aggregate is 0.5, 1%. Use 20mm size aggregate. | 10M | CO-3 | BL-4 |

***