| Subject Code 22PC1EC211 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year I Semester Regular/Supplementary Examinations, December, 2024

DIGITAL LOGIC DESIGN

(Common to CSE, IT, CyS, and IoT)

Time: 3 hours                                                     			      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A

5X2=10M

|  | Convert i) (10110)2 to gray code ii) Gray code (110101) to binary number. | 2M | CO-1 | BL-3 |
| --- | --- | --- | --- | --- |
|  | Derive the sum of minterms for the function f(a,b,c)=a′b+b′c′ | 2M | CO-2 | BL-3 |
|  | Define a combinational logic circuit and draw its block diagram. | 2M | CO-3 | BL-1 |
|  | Differentiate between Latch and Flip Flop. | 2M | CO-4 | BL-3 |
|  | Write the differences between synchronous and asynchronous counters. | 2M | CO-5 | BL-2 |

PART-B

5X10=50M

| UNIT-1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1. | a) Given the 8 bit data word 11011011. Generate the 12 bit composite word for the Hamming code that corrects and detects single error. |  | 5M |  | CO-1 |  | BL-3 |
|  | b) What is the use of complements? Perform subtraction using 10’s complement for (72532)-(3250). |  | 5M |  | CO-1 |  | BL-2 |
| OR |  |  |  |  |  |  |  |
| 2. | a) Convert Hexadecimal number (6BA9)16 to base 2 and base 8 number systems. |  | 5M |  | CO-1 |  | BL-3 |
|  | b) Explain about even and odd parity check with an example. |  | 5M |  | CO-1 |  | BL-2 |
| UNIT-II |  |  |  |  |  |  |  |
| 3. | a) Obtain the simplified expression in sum of products for the following Boolean function. F(A,B,C,D) = ∑(2,3,12,13,14,15). |  | 5M |  | CO-2 |  | BL-3 |
|  | b) Derive and Implement Exclusive-OR function involving three variables using only NAND gates. |  | 5M |  | CO-2 |  | BL-2 |
| OR |  |  |  |  |  |  |  |
| 4. | a) Simplify the Boolean expression using K-map F(A,B,C,D) = Σm(1,2,3,8,9,10,11,14) +d(7,15). |  | 3M |  | CO-2 |  | BL-3 |
|  | b) Simplify the following Boolean function using the tabulation method by determining the prime implicants F(w, x, y, z) = ∑m(1, 4, 6, 7, 8, 9, 10, 11, 15) and verify it with K-map. |  | 7M |  | CO-2 |  | BL-3 |
| UNIT-III |  |  |  |  |  |  |  |
| 5. | - Define a multiplexer? Draw a 4:1 multiplexer for the function f(a,b,c,d)=∑m(0, 4,5,10,11,12,15). |  | 5M |  | CO-3 |  | BL-2 |
|  | - Design a full binary adder with two half adders and a OR gate. |  | 5M |  | CO-3 |  | BL-3 |
| OR |  |  |  |  |  |  |  |
| 6. | A Combinational circuit is defined by the following three Boolean functions F1= x′y′z′ + xz F2 = yx ′z′ + x′y F3 = x′y′z + xy Design the circuit with a decoder and external gates. | 10M |  | CO-3 |  | BL-3 |  |
| UNIT-IV |  |  |  |  |  |  |  |
| 7. | - Convert a SR flip flop to D type flip flop? |  | 5M |  | CO-4 |  | BL-3 |
|  | - Draw and explain the block diagram of PLA with a simple example. |  | 5M |  | CO-4 |  | BL-2 |
| OR |  |  |  |  |  |  |  |
| 8. | What is the drawback of JK flip flop, design a flip flop which overcomes this drawback and explain with neat diagram. | 10M |  | CO-4 |  | BL-2 |  |
| UNIT-V |  |  |  |  |  |  |  |
| 9. | a) Draw a 4-bit Asynchronous up counter using JK flip flops. |  | 3M |  | CO-5 |  | BL-2 |
|  | b) Design a sequence detector that detects the binary sequence 1101 using a Mealy machine. Provide the state diagram and explain the transitions. |  | 7M |  | CO-5 |  | BL-3 |
| OR |  |  |  |  |  |  |  |
| 10. | a) Design MOD-8 synchronous counter with T flip flops. |  | 5M |  | CO-5 |  | BL-2 |
|  | - Explain different types of shift registers. |  | 5M |  | CO-5 |  | BL-2 |

***