| Subject Code 22PC1EE201 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year I Semester Regular/Supplementary Examinations, December, 2024

ELECTRICAL MACHINES - I

(EEE)

Time: 3 hours                                                                                 		    Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | What is the purpose of commutator in a dc motor? | 2M | CO-2 | BL-1 |
| --- | --- | --- | --- | --- |
|  | What are the types of DC generators? | 2M | CO-2 | BL-2 |
|  | Which motor is best suitable for traction application? Justify? | 2M | CO-3 | BL-3 |
|  | Write about ideal transformer? | 2M | CO-1 | BL-2 |
|  | Write different types of three phase transformers. | 2M | CO-2 | BL-2 |

PART-B				         5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | - Explain the differences between the lap and wave windings in DC Machine? | 5M | CO-1 | BL-2 |
|  | - Derive the EMF equation of a DC Generator? | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 2. | Explain working principle and action of commutator in DC Generator? | 10M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |
| 3. | - Explain about OCC characteristics of DC Shunt generator? | 5M | CO-2 | BL-2 |
|  | b) A 4 pole lap wound DC shunt generator delivers 200 A at terminal voltage of 250 V. It has a field and armature resistance of 50 Ω and 0.05 Ω respectively. Neglecting brush drop determine (i) armature current (ii) EMF generated (iii) power delivered. | 5M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 4. | Explain about armature reaction and also derive the cross and demagnetizing ampere turns per pole in DC generator? | 10M | CO-3 | BL-2 |
| UNIT-III |  |  |  |  |
| 5. | - Derive the torque equation of DC motor? | 5M | CO-2 | BL-2 |
|  | b) A 250-volt DC shunt motor has armature resistance of 0.25 ohm, on load it takes an armature current of 50 A and runs at 750 r.p.m. If the flux of motor is reduced by 10% without changing the load torque, find the new speed of the motor. | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 6. | Analyse the Swinburne’s test on DC shunt motor in detail. | 10M | CO-2 | BL-4 |
| UNIT-IV |  |  |  |  |
| 7. | - Derive the emf equation of transformer? | 5M | CO-1 | BL-2 |
|  | - Explain Short circuit test on single phase transformer? | 5M | CO-2 | BL-4 |
| OR |  |  |  |  |
| 8. | A 10KVA, 4800V/240V,60Hz, single phase transformer gave the following test results. No load test (LV); 240V; 1.5A; 150watt. Short circuit test (HV); 165V; 1.8A; 170Watts. Calculate the (i) No load power factor, magnetizing current and iron loss component current at normal voltage and frequency (ii) Core loss and full load copper loss (iii) obtain the equivalent circuit on LV and HV sides (iv) Find the efficiency at half load with 0.8 p.f. lagging | 10M | CO-4 | BL-3 |
| UNIT-V |  |  |  |  |
| 9. | - Explain about the advantages of auto transformer over single phase transformer? | 5M | CO-3 | BL-2 |
|  | b) A 10 kVA, 230/110 V transformer is to be used as step-up transformer. What will be the voltage ratio and transformation ratio of step-up transformer? | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 10. | Explain parallel operation of single-phase transformer. | 10M | CO-3 | BL-2 |

***