| Subject Code 22PC1EE202 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year I Semester Regular/Supplementary Examinations, December, 2024

ELECTROMAGNETIC FIELDS

(EEE)

Time: 3 hours                                                      			      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A

5X2=10M

|  | State coulombs law of electric force. | 2M | CO-1 | BL1 |
| --- | --- | --- | --- | --- |
|  | Define electric dipole. | 2M | CO-1 | BL1 |
|  | Derive a relation between Magnetic field Intensity and magnetic flux Density | 2M | CO-2 | BL2 |
|  | What is the significance of Lorentz force equation? | 2M | CO-3 | BL1 |
|  | What is displacement current? Differentiate conduction current and displacement current? | 2M | CO-4 | BL2 |

PART-B

5X10=50M

| UNIT-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | - Find E at a point (4, -3, 2) m due to a point charge of 64.4 nC located at the origin in Cartesian coordinates. | 5M | CO-1 | BL-1 |
|  | - Obtain Poisson’s and Laplace equations from Gauss’s law. | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 2. | Derive an expression for Electric field intensity due to infinite line charge distribution? | 10M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |
| 3. | - Derive Equation of continuity in point form. What is its significance? | 5M | CO-2 | BL-3 |
|  | - Derive the expressions for the capacitance of a parallel plate capacitor and the energy stored in it. | 5M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 4. | Derive and state the electrostatic boundary conditions between two perfect dielectrics. | 10M | CO-2 | BL-3 |
| UNIT-III |  |  |  |  |
| 5. | - A steady current element of 10-3az Am is located at the origin in free space. Find the magnetic field Intensity at the point (1,0,0) m. | 5M | CO-3 | BL-3 |
|  | - State the Ampere’s circuital law and Derive the Point form of Ampere’s circuital law. | 5M | CO-3 | BL-2 |
| OR |  |  |  |  |
| 6. | State Biot-savart’s Law and Derive an expression for magnetic field intensity due to an infinite long straight current carrying conductor. | 10M | CO-3 | BL-2 |
| UNIT-IV |  |  |  |  |
| 7. | - Two infinitely long parallel conductors are separated by a distance ‘d’. Find the force per unit length exerted by one of the conductors on the other if the currents in the two conductors are I1 and I2. | 5M | CO-4 | BL-3 |
|  | - Derive the expression for energy stored in a magnetic field. | 5M | CO-4 | BL-3 |
| OR |  |  |  |  |
| 8. | The point charge Q=18nc has a velocity of 5 x 106 m/s in the direction av=0.6ax+0.75 ay+.3az. Calculate the magnitude of the force exerted on the charge by the field: a) B=-3ax+4 ay+6az mT b) E=-3ax+4 ay+6az kV/m c) B and E both are acting together. | 10M | CO-4 | BL-3 |
| UNIT-V |  |  |  |  |
| 9. | - State the Poynting Theorem and derive the necessary expressions. | 5M | CO-4 | BL-3 |
|  | - Determine the inductance of a solenoid of 2500 turns wound uniformly over a length of 0.25m on a cylindrical paper tube, 4 cm in diameter and the medium is air. | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 10. | Write Maxwell’s equation for static fields. Explain how they are modified for time varying electric and magnetic fields. | 10M | CO-4 | BL-2 |

****