| Subject Code 22PC1EC201 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year I Semester Regular/Supplementary Examinations, December, 2024

ELECTRONIC CIRCUIT ANALYSIS

(ECE)

Time: 3 hours                                                                  Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A

5X2=10M

|  | Define four h parameters of BJT model and draw it in CE configuration. | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | What are the different coupling schemes in multistage amplifiers? | 2M | CO-2 | BL-1 |
|  | Define positive and negative feedback. | 2M | CO-3 | BL-2 |
|  | Compare RC and LC oscillators. | 2M | CO-4 | BL-3 |
|  | Draw the small signal model of CS amplifier. | 2M | CO-5 | BL-1 |

PART-B

5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | - A CE amplifier is drawn by a voltage source of internal resistance rs = 700 Ω, and the load impedance is a resistance RL = 1000Ω. The h-parameters are hie=1KΩ, hre=2x10-4, hfe=50 and hoe=25μA/V. Compute the current gain, input resistance, voltage gain and output resistance. | 5M | CO-1 | BL-4 |
|  | - What is the effect of coupling and bypass capacitors on CE amplifiers. | 5M | CO-1 | BL-3 |
| OR |  |  |  |  |
| 2. | Derive the expressions for voltage gain, current gain, input impedance and output impedance of CE amplifier with emitter resistance. | 10M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |
| 3. | - What is the need for coupling? Discuss the RC coupled amplifier and its frequency response. | 5M | CO-2 | BL-2 |
|  | - Discuss the CE amplifier at high frequencies and derive the short circuit current gain. | 5M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 4. | - Justify, the Darlington amplifier suitable for impedance matching and current amplification. | 5M | CO-2 | BL-4 |
|  | - Discuss the frequency response of single stage CE amplifier at high frequencies | 5M | CO-2 | BL-3 |
| UNIT-III |  |  |  |  |
| 5. | - Derive the expressions for Avf, Rif, and Rof for voltage series feedback amplifier. | 5M | CO-3 | BL-3 |
|  | - An amplifier has a gain of 2*105 without feedback. Determine the gain if negative feedback is applied. - Given β =0.02 - Given β =0.04 - If Without feedback amplifier gain is 4*105 | 5M | CO-3 | BL-4 |
| OR |  |  |  |  |
| 6. | - Derive the expressions for voltage gain of an amplifier with positive and negative feedback and compare closed loop and open loop voltage gain. | 5M | CO-3 | BL-3 |
|  | - Derive the expressions for Avf, Rif, and Rof for current series feedback amplifier. | 5M | CO-3 | BL-3 |
| UNIT-IV |  |  |  |  |  |
| 7. | Derive the expression for frequency of oscillations and the condition for starting oscillations in Hartley oscillator. | 10M | CO-4 | BL-3 |
| OR |  |  |  |  |
| 8. | - Summarize the Barkhausen’s criteria conditions for sustained oscillation. | 5M | CO-4 | BL-4 |
|  | - Given that L1=1000μH, L2=100μH, the mutual inductance between the coils M = 20 μH, C=20PF. Calculate the (i) operating frequency and (ii) feedback factor for Hartley oscillator. | 5M | CO-4 | BL-4 |
| UNIT-V |  |  |  |  |
| 9. | - Analyse common source JFET amplifier to find voltage gain. | 5M | CO-5 | BL-3 |
|  | - Discuss Common source amplifier with resistive load. | 5M | CO-5 | BL-3 |
| OR |  |  |  |  |
| 10. | - Design and analyse the source follower amplifier. | 5M | CO-5 | BL-4 |
|  | - Discuss the concept of MOSFET amplifiers. | 5M | CO-5 | BL-3 |

***