| Subject Code 22PC1CE203 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year I Semester Regular/Supplementary Examinations, December/January 2025

FLUID MECHANICS

(CE)

Time: 3 hours                                                                                 		       Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A				5X2=10M

|  | Differentiate between mechanical gauges and manometers. | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | Define the terms (i) Velocity Potential function (ii) Stream function | 2M | CO-1 | BL-1 |
|  | Define velocity of approach. What is the effect of velocity of approach on the discharge through a weir? | 2M | CO-2 | BL-2 |
|  | Enumerate various losses in pipe flow. | 2M | CO-1 | BL-1 |
|  | What is meant by boundary layer separation and how it can be prevented? | 2M | CO-1 | BL-2 |

PART-B				5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | - State the ‘Pascal’s Law’ and explain with help of ram and plunger. | 5M | CO-2 | BL-2 |
|  | - A Cylinder of 0.3 m diameter rotates concentrically inside a fixed cylinder of 0.31 m diameter. Both the cylinders are 0.3 m long. Determine the viscosity of the liquid which fills the space between the cylinders if a torque of 0.9 N-m is required to maintain an angular velocity of 2 rad/sec | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 2. | a) Explain the relationship among different pressures using a neat diagram. | 5M | CO-2 | BL-3 |
|  | b) A mercury manometer is used to determine the pressure difference at two different points in a pipe carrying oil of specific gravity 0.8. The pipe connected on the right limb of the manometer is 5 cm above the level of mercury in the right limb. The pipe connected to the left limb is 3 cm below the center of pipe on the right. If the difference in mercury levels in both the limbs is 8 cm with the level higher in the right limb, determine the pressure difference in the two limbs. | 5M | CO-3 | BL-3 |
| UNIT-II |  |  |  |  |
| 3. | - Explain the classification of fluid flows. | 5M | CO-1 | BL-2 |
|  | b) In a two dimensional flow field for an incompressible fluid the velocity components are; u = y3/3 + 2x – x2y; v = xy2 – 2y – x3. Determine an expression for the stream function ‘’ | 5M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 4. | a) State and explain Bernoulli’s equation. | 5M | CO-2 | BL-2 |
|  | b) A pipe line carrying oil of specific gravity 0.87, changes in diameter from 200 mm diameter at point A to 500 mm diameter at point B, which is 4 meters at a higher level. If the pressure at A and B are 15 N/cm2 and 10.4 N/cm2 respectively and the discharge is 200 lts/sec, determine the loss of head and direction of flow | 5M | CO-3 | BL-3 |
| UNIT-III |  |  |  |  |
| 5. | - Explain the principle of Venturimeter with neat sketch. Derive an equation for the rate of flow through it. | 5M | CO-2 | BL-2 |
|  | - Water flows over a rectangular weir 1 m wide at a depth of 150 mm and afterwards passes through a triangular right angled weir. Find the depth over the triangular weir. Take Cd= 0.62 for rectangular weir and 0.60 for triangular weir | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 6. | a) Derive hydraulic co-efficients of an orifice with neat figure. | 6M | CO-2 | BL-2 |
|  | b) An orifice meter with orifice diameter 15 cm is inserted in a pipe of 30 cm diameter. The pressure difference measured by a mercury oil differential manometer on the two sides of the orifice meter gives a reading of 50 cm of mercury. Determine the rate of flow of oil of sp. Gr.0.9, when the coefficient of discharge of the meter is 0.64 | 4M | CO-3 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7. | Derive an expression for loss of energy due to friction in a pipe flow (Darcy-Weish equation). | 10M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 8. | a) Describe Reynolds experiment to demonstrate the types of flow | 5M | CO-1 | BL-2 |
|  | b) In a circular pipe of diameter 100 mm, a fluid of viscosity 7 poise and specific gravity 1.3 is flowing. If the maximum shear stress at the wall of the pipe is 196.2 N/m2. Determine i) The pressure gradient ii) The average velocity iii) Reynolds of the flow | 5M | CO-3 | BL-3 |
| UNIT-V |  |  |  |  |
| 9. | a) Define: i) Laminar boundary layer ii) Turbulent boundary layer iii) Boundary layer thickness | 5M | CO-1 | BL-2 |
|  | b) Determine the momentum thickness and energy thickness for the velocity distribution in the boundary layer given by u/U = 2 (y/) –(y/)2 | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 10. | a) What do you mean by separation of boundary layer? Discuss the effect of pressure gradient on boundary layer separation | 5M | CO-1 | BL-2 |
|  | b) A flat plate 2mx2m moves at 40 km/hour in stationary air of density 1.25 kg/m3. If CD and CL are 0.2 and 0.8, respectively, find i) Drag force ii) Lift Force iii) Resultant Force iv) Power required to keep the plate in motion. | 5M | CO-3 | BL-3 |

***