| Subject Code 22PC1AE205 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year I Semester Regular/Supplementary Examinations, December/January 2025

FUNDAMENTALS OF THERMODYNAMICS

(AE)

Time: 3Hrs										      Max.Marks:60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | Differentiate the microscopic & macroscopic study. | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | Define specific heat at constant pressure and constant volume. | 2M | CO-1 | BL-1 |
|  | State the Kelvin plank and Clausius statements of second laws. | 2M | CO-1 | BL-1 |
|  | Draw PV diagram for pure substance. | 2M | CO-2 | BL-2 |
|  | List the thermodynamic processes involved in the simple Brayton Cycle. | 2M | CO-3 | BL-2 |

PART-B				5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) Explain the differences between open, closed, and isolated systems with examples. Analyze the thermodynamic behavior of a coffee cup in a room. | 5M | CO-1 | BL-3 |
|  | b) Differentiate between intensive and extensive properties with suitable examples. Identify whether pressure, volume, density, and specific energy are intensive or extensive. | 5M | CO-1 | BL-3 |
| OR |  |  |  |  |
| 2. | a) How will the constant volume gas thermometer measure the temperature? | 5M | CO-1 | BL-3 |
|  | b) Derive the expression for displacement work in a polytropic process. | 5M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |
| 3. | a) State and explain the First Law of Thermodynamics for a closed system. | 5M | CO-1 | BL-2 |
|  | b) A piston-cylinder contains 2 kg of air at 500 kPa and 400C. Heat is added at constant pressure until the temperature reaches 200°C. Determine the work done and heat transfer, assuming no change in kinetic or potential energy. | 5M | CO-1 | BL-3 |
| OR |  |  |  |  |
| 4. | Derive the steady flow energy equation for flow process. | 10M | CO-2 | BL-3 |
| UNIT-III |  |  |  |  |
| 5. | a) A Carnot engine operates between a heat source at 600 K and a sink at 300 K. The engine delivers 10 kW of power. Calculate the heat supplied and rejected. | 5M | CO-2 | BL-3 |
|  | b) Derive the absolute thermodynamic temperature scale. | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 6. | Derive the expression for entropy change for an ideal gas undergoing an isothermal process. Use this to calculate the entropy change when 2 kg of air expands isothermally at 400 K from 200 kPa to 100 kPa. | 10M | CO-2 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7. | Define a pure substance and explain how it differs from a mixture. Explain the measurement of steam quality with throttling calorimeter. | 10M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 8. | A mixture of gases contains 2 kg of nitrogen and 4 kg of oxygen in a 1 m³ tank at 300 K. Assuming ideal gas behavior: - Calculate the partial pressures of each gas. - Determine the total pressure in the tank. - Find the average molecular weight of the mixture. | 10M | CO-2 | BL-4 |
| UNIT-V |  |  |  |  |
| 9. | Define and list the causes of irreversibility in thermodynamic processes. Provide examples of processes with high irreversibility and their implications on system efficiency. | 10M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 10. | Show the Otto cycle on p-v and T-s. diagrams and derive the expression for thermal efficiency. | 10M | CO-3 | BL-3 |

****