| Subject Code 22PC1ME204 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year I Semester Regular/Supplementary Examinations, December/January 2025

KINEMATICS OF MACHINERY

(ME)

Time: 3 hours                                                                              		Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | Distinguish between mechanism and machine. | 2M | CO 1 | BL2 |
| --- | --- | --- | --- | --- |
|  | How do you find the line of action, magnitude and direction of the velocity of a point on a link | 2M | CO 2 | BL2 |
|  | Discuss the merits and demerits of Davis or Ackermann steering gear mechanism. | 2M | CO 4 | BL2 |
|  | Draw the displacement and velocity variation of the ascent motion of follower when it moves with uniform velocity. | 2M | CO 3 | BL2 |
|  | What is meant by a gear train and mention an application. | 2M | CO 3 | BL1 |

PART-B				        5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) Explain any two inversions of four bar chain with applications. | 5M | CO 1 | BL-2 |
|  | b) In a slider crank mechanism, the driving crank length is 60 mm. The connecting rod length is 240 mm. Determine the stroke length of the sliding piston. | 5M | CO 1 | BL-3 |
| OR |  |  |  |  |
| 2. | - Explain an inversion of double slider crank chain with neat sketch. | 5M | CO 1 | BL-2 |
|  | - Explain different types of constrained motion. | 5M | CO 1 |  |
| UNIT-II |  |  |  |  |
| 3. | In a working 4 bar mechanism, draw two different positions of the mechanism, when the driving crank rotates by an angle in the counter clockwise direction. | 10M | CO 2 | BL-4 |
| OR |  |  |  |  |
| 4. | In a slider crank mechanism the crank is 220 mm long and rotates at 30 rad/sec in a CCW direction. The length of the connecting rod is 850 mm. When the crank turns through 60o from Inner-dead centre. Determine (i) the velocity and acceleration of the slider(ii) Velocity and acceleration of point E located at a distance of 250 mm on the connecting rod. | 10M | CO 2 | BL-3 |
| UNIT-III |  |  |  |  |
| 5. | a) Sketch the peaucellier mechanism and show that the tracing point describes a straight line. | 5M | CO 4 | BL-2 |
|  | b) Mention the velocity ratio of two shafts connected by a Hooke’s Joint, if the angular velocities are w1 and w. The shafts are at angle Alpha. Two shafts are connected by a Hooke’s joint. The driving shaft revolves uniformly at 150 r.p.m. If the total permissible variation in speed of a driven shaft is not to exceed ±6% of the mean speed, find the greatest permissible angle between the centre lines of the shafts. | 5M | CO 4 | BL-3 |
| OR |  |  |  |  |
| 6. | - Sketch a pantograph and show that it satisfies the required conditions | 5M | CO 4 | BL-2 |
|  | - Draw and explain Hookes joint with its applications | 5M | CO 4 | BL-2 |
| UNIT-IV |  |  |  |  |
| 7. | The following data relates to a rotating cam reciprocating follower in which the follower moves with uniform acceleration and deceleration during the outstroke and with uniform velocity during the return stroke. Stroke length = 28 mm, Angle of outstroke = 60o, Angle of return stroke = 90o, Angle of dwell between outstroke and return stroke = 45o, Speed of the cam = 200 rpm. Draw the trends of the displacement, velocity, and acceleration. Also mention the formulae for the maximum velocity and the uniform acceleration of the follower during the outstroke and the return stroke. | 10M | CO 3 | BL-3 |
| OR |  |  |  |  |
| 8. | Draw the trends of the kinematic parameters in a rotating cam and oscillating follower from the following data. a) 120 0 rotation of cam during outstroke with Simple Harmonic Motion. b) The follower remains at rest for the next 700rotation of the cam. c)The follower then returns to its original position during 1000 rotation of the cam with Simple Harmonic Motion. d) The follower remains at rest for the rest of the revolution. The path of the follower may be assumed. Mention the formulae for the kinematic parameters. If the cam rotates at 3000 RPM, show the substitution. | 10M | CO 3 | BL-3 |
| UNIT-V |  |  |  |  |
| 9. | Two mating involute spur gears of 20o pressure angle have a gear ratio of 2. The number of teeth on the pinion is 20 and its speed is 250 rpm. The module pitch of the teeth is 12 mm if the addendum on each wheel is such that the path of approach and the path of recess on each side are 40% of the maximum possible length each, find (a) the addendum for pinion and gear wheel (b) the length of arc of contact (c) the maximum velocity of sliding during approach and recess. Assume pinion to be driver. | 10M | CO 3 | BL-3 |
| OR |  |  |  |  |
| 10. | a) Explain any two types of gear trains with neat sketches b) Describe the procedure to analyze epi cyclic gear train. | 5M 5M | CO 3 CO3 | BL-2 BL-2 |

***