| Subject Code 22PC1AE204 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year I Semester Regular/Supplementary Examinations, December/January 2025

MECHANICS OF MATERIALS

(AE)

Time: 3 hours                                                                                 			     Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | Define Poisson’s Ratio. | 2M | CO-1 | BL-I |
| --- | --- | --- | --- | --- |
|  | What are the different types of beams? Differentiate between a cantilever and a simply ported beam. | 2M | CO-3 | BL-2 |
|  | Write the theory of simple bending equation with usual notation. | 2M | CO-3 | BL-I |
|  | Define principal stresses. | 2M | CO-2 | BL-I |
|  | Find the slope, of a simply supported beam of span L, carrying a central point load P. | 2M | CO-3 | BL-2 |

PART-B					5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | - Draw the stress-strain curve for mild steel, highlighting its key features. | 5M | CO-1 | BL-1 |
|  | b) A rod with a length of 200 cm and a diameter of 3.0 cm is subjected to an axial load of 30 kN. If the modulus of elasticity for the material of the rod is 1.8×105 N/mm2, calculate the following: i) The stress on the rod. ii) The strain in the rod. iii) The elongation of the rod. | 5M | CO-1 | BL-3 |
| OR |  |  |  |  |
| 2. | An aluminum bar with a cross-sectional area of 1500 mm² is subjected to axial forces as shown in the figure. The Young's modulus for aluminum typically ranges from 70 GPa. Determine the total elongation of the bar. | 10M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |
| 3. | - Deduce the relation between shear force and rate of loading. | 3M | CO-3 | BL-4 |
|  | - A simply supported beam has a span of 9 m and carries a uniformly distributed load of 15 kN/m over the whole span along with two point loads of 20 kN and 40 kN at 4 m and 6 m respectively from the left hand support. Draw the shear force and bending moment diagrams indicating the values at the point loads. | 7M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 4. | A hollow shaft is required to transmit 600 kW at 110 r.p.m., the maximum torque being 20% greater than the mean. The shear stress is not to exceed 63 MPa and twist in a length of 3 metres not to exceed 1.4 degrees. Find the external diameter of the shaft, if the internal diameter to the external diameter is 3/8. Take modulus of rigidity as 84 GPa. | 10M | CO-1 | BL-3 |
| UNIT-III |  |  |  |  |
| 5. | - Find the ratio of maximum shear stress to average shear stress in case of a rectangular section of size bXd. | 5M | CO-3 | BL-3 |
|  | - The cross section of joint is a T section 150 mm x 100 mm x 13 mm with 150 mm side horizontal. Find the maximum intensity of shear stress and sketch the distribution of stress across the section, if it has to resist a shear force of 45 MPa. | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 6. | A simple beam AB of span length 6.7 m supports a uniform load of intensity q = 22 kN/m and a concentrated load p = 50 kN. The uniform load includes an allowance for the weight of the beam. The concentrated load acts at a point 2.5 m from the left-hand end of the beam. The beam is constructed of glued laminated wood and has a rectangular cross section of width b = 220 mm and a height h = 700 mm. Determine the maximum tensile and compressive stresses in the beam due to bending. | 10M | CO-3 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7. | A cylindrical shell 3 m long, 1 m internal diameter and 10 mm thick is subjected to an internal pressure of 1.5 N/mm2. Calculate the changes in length, diameter and volume of the cylinder. E=200 KN/mm2, Poisson’s ratio = 0.3. | 10M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 8. | A solid shaft with a diameter of 90 mm is subjected to a twisting moment of 10 MN-mm and a bending moment of 6 MN-mm at a certain point. Determine: (a) The principal stresses (b) The orientation of the plane on which these principal stresses act. | 10M | CO-2 | BL-3 |
| UNIT-V |  |  |  |  |
| 9. | A cantilever beam of length 5 meters carries a uniformly varying load with zero intensity at the free end and 25 kN/m at the fixed end. Given that the modulus of elasticity E = 2.2X105 N/mm2 and the moment of inertia I = 1.2 X 108 mm4, calculate the slope and deflection at the free end. | 10M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 10. | a) Explain the concept of effective length of a column for different end conditions. | 3M | CO-3 | BL-2 |
|  | b) A column with one end fixed and the other end free has an actual length of 4 m. If the cross-section is circular with a diameter of 40 mm, calculate the critical load using Euler’s formula. Take E = 210 GPa. | 7M | CO-3 | BL-3 |

***