| Subject Code 22PC1ME201 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year I Semester Regular/Supplementary Examinations, December, 2024

MECHANICS OF SOLIDS

(ME)

Time: 3 hours                                                                                 			      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A

5X2=10M

|  | Define the term bars of varying sections. | 2M | CO1 | BL1 |
| --- | --- | --- | --- | --- |
|  | What are the different types of loads? | 2M | CO2 | BL1 |
|  | Sketch the shear stress distribution across the depth of ‘I ‘section. | 2M | CO3 | BL2 |
|  | What is Mohr’s circle? Write its significance. | 2M | CO4 | BL1 |
|  | Write advantages of Macaulay’s method for finding deflection of beams? | 2M | CO5 | BL2 |

PART-B

5X10=50M

| UNIT-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | Find the extension of the bar shown in Fig. under an axial load of 25 kN. | 10M | CO-1 | BL-3 |
| OR |  |  |  |  |
| 2 | A steel rod of 3 cm diameter and 5 m long is connected to two grips and the rod is maintained at a temperature of 95°C. Determine the stress and pull exerted when the temperature falls to 30° C, if - the ends do not yield, and - the ends yield by 0.12 cm. Take E=2x 105 MN/m2 and α = 12x 10-6/°C. | 10M | CO-1 | BL-4 |
| UNIT-II |  |  |  |  |
| 3 | A cantilever beam of length 5 m carries the point loads as shown in Fig. Draw the shear force and B.M. diagrams for the cantilever beam. | 10M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 4 | A simply supported beam of length 6 m, carries point load of 3 kN and 6 kN at distances of 2 m and 4 m from the left end. Draw the shear force and bending moment diagrams for the beam. | 10M | CO-2 | BL-3 |
| UNIT-III |  |  |  |  |
| 5 | A rectangular beam 250 mm deep is simply supported over a span of 6 m. Determine the uniformly distributed load per metre which the beam may carry, if the bending stress should not exceed 140 N/mm2. Take I = 8 x 106 mm4 | 10M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 6 | A beam of I-section shown in Fig is simply supported over a span of 10 m. It carries a uniform load of 4 kN/m over the entire span. Evaluate the maximum bending stresses. | 10M | CO-3 | BL-4 |
| UNIT-IV |  |  |  |  |
| 7 | An elemental cube is subjected to tensile stresses of 30 N/mm2 and 10 N/mm2 acting on two mutually perpendicular planes and a shear stress of 10 N/mm2 on these planes. Determine the magnitudes and directions of principal stresses and also the greatest shear stress. | 10M | CO-4 | BL-4 |
| OR |  |  |  |  |
| 8 | Calculate the change in diameter, change in length, the change in volume of a thin cylindrical shell 100 cm diameter, 1 cm thick and 5m long when subjected to internal pressure of 3 N/mm2 and Poisson’s ratio of 0.3. | 10M | CO-4 | BL-3 |
| UNIT-V |  |  |  |  |
| 9 | A beam of length 6m is simply supported at ends and carries two-point loads of 48 kN and 40 kN at distance of 1m and 3m respectively from the left support. Using Macauley’s method find (i) deflection under each load (ii) maximum deflection (iii) the point at which maximum deflection occurs. - Given, E = 2 x 105 N/mm2 & I = 85 x 106 mm4 | 10M | CO-5 | BL-4 |
| OR |  |  |  |  |
| 10 | A beam of uniform rectangular section 200 mm wide and 300 mm deep is simply supported at its ends. It carries a uniformly distributed load of 9 KN/m run over the entire span of 5 m. if the value of E for the beam material is 1 X 104 N/mm2, find the slope at the supports and maximum deflection. | 10M | CO-5 | BL-3 |

****