| Subject Code 22PC1EC203 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year I Semester Regular/Supplementary Examinations, December, 2024

SIGNALS AND SYSTEMS

(Common to ECE and EIE)

Time: 3 hours                                                                                 		    Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A				           5X2=10M

|  | Find the even and odd components of the signal x(t)= ej2t. | 2M | CO1 | BL3 |
| --- | --- | --- | --- | --- |
|  | State Dirichlet conditions for the existence of Fourier series. | 2M | CO2 | BL1 |
|  | Find the Inverse Laplace transform of 1(s+a)2. | 2M | CO2 | BL3 |
|  | State Parsevals theorem. | 2M | CO4 | BL1 |
|  | Find the z – transform of x[n – n0] | 2M | CO2 | BL3 |

PART-B				         5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | - Determine the average power and normalized energy of the signal x(t) = A sin (ω0t+θ). | 5M | CO1 | BL3 |
|  | - Determine whether the following function is periodic or not. If, so find the period. x(t) = 3 sin 200 πt + 4 cos 100t. | 5M | CO1 | BL3 |
| OR |  |  |  |  |
| 2. | Distinguish between the following: i) Continuous time signal and discrete time signal. ii) Unit step and Unit Ramp functions. iii) Periodic and Aperiodic Signals. iv) Deterministic and Random Signals. | 10M | CO1 | BL2 |
| UNIT-II |  |  |  |  |
| 3. | - Derive the expression for Fourier Transform from Fourier Series. | 5M | CO2 | BL3 |
|  | - What is the significance of Hilbert Transform? Give the mathematical analysis. | 5M | CO2 | BL1 |
| OR |  |  |  |  |
| 4. | Find the Exponential Fourier series and plot the magnitude and phase spectra of the following triangular wave form. | 10M | CO2 | BL4 |
| UNIT-III |  |  |  |  |
| 5. | - Discuss various properties of ROC of Laplace transform. | 5M | CO2 | BL2 |
|  | - Find the inverse Laplace transform for the following: i) Xs= 2S+1(S+1)(S2+2S+2) ii) XS= 4s2+8S+10(S+2)(S2+2S+5) | 5M | CO2 | BL3 |
| OR |  |  |  |  |
| 6. | Obtain the convolution of the following two functions using graphical method. xt=1;for-3≤t≤3 and ht=2;for 0≤t≤3 | 10M | CO4 | BL4 |
| UNIT-IV |  |  |  |  |
| 7. | Obtain the condition for distortionless transmission through a system. Also show that for linear phase systems, the impulse response is symmetrical about t =td.. | 10M | CO3 | BL2 |
| OR |  |  |  |  |
| 8. | a) Define auto correlation function and explain its properties. | 5M | CO4 | BL2 |
|  | b) With an example explain how correlation can be used for the extraction of signal in the presence of noise. | 5M | CO4 | BL2 |
| UNIT-V |  |  |  |  |
| 9. | State and prove the sampling theorem for Band limited signals. | 10M | CO5 | BL2 |
| OR |  |  |  |  |
| 10. | a) Find the inverse z-transform of X(z) = 11-az-1 with ROC ¦z¦ < ¦a¦ | 5M | CO2 | BL3 |
|  | b) State and prove the differentiation in z -domain property of z-transform. | 5M | CO2 | BL3 |

***