| Subject Code 22PC1CE201 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year I Semester Regular/Supplementary Examinations, December, 2024

STRENGTH OF MATERIALS-I

(CE)

Time: 3Hrs											      Max.Marks:60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | Define elasticity and plasticity? | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | What is the difference between Resilience and Proof Resilience? | 2M | CO-1 | BL-1 |
|  | What is the difference between simply supported beam and cantilever beam? | 2M | CO-2 | BL-1 |
|  | What are the assumptions made in theory of simple bending? | 2M | CO-3 | BL-1 |
|  | Define bending and shear stress. | 2M | CO-4 | BL-2 |

PART-B					5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | A steel rod of 3 cm diameter is enclosed centrally in a hollow copper tube of external diameter 5cm and internal diameter of 4cm. the composite bar is then subjected to an axial pull of 45000N.if the length of each bar is equal to 15 cm, determine: i) The stresses in the rod and tube, and (ii) Load carried by each bar Take (E) for steel=2.1x105 N/mm2 and for copper = 1.1x105 N/mm2. | 10M | CO-1 | BL-4 |
| OR |  |  |  |  |
| 2. | Determine the changes in length, breadth and thickness of a steel bar which is 4 m long, 30 mm wide and 20 mm thick and is subjected to axial pull of 30 KN in the direction of its length. Take E=2x105N/mm2 and μ=0.3. | 10M | CO-1 | BL-4 |
| UNIT-II |  |  |  |  |
| 3. | Define Strain energy. A tensile load of 60 KN is gradually applied to a circular bar of 4 cm diameter and 5 m long if E=2x105N/mm2 . Determine: i) stretch in the rod (ii) stress in the rod and (iii) strain energy absorbed by the rod. | 10M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 4. | A compound tube consists of a steel tube 140mm internal diameter and 160mm external diameter and an out brass tube 160mm internal diameter and 180mm external diameter. The two tubes are of the same length. The compound tube carries an axial load of 900 kN. Find the stresses and the load carried by each tube. Length of each tube is 140mm. Take E for Steel as 2 x 105 N/mm2 and for brass as 1 x 105 N/mm2. | 10M | CO-1 | BL-4 |
| UNIT-III |  |  |  |  |
| 5. | A cantilever of length 3 m carries a uniformly distributed load of 2.5 KN/m length over the whole length and a point of 3.5 KN at the free end. Draw SFD and BMD for the cantilever. | 10M | CO-2 | BL-4 |
| OR |  |  |  |  |
| 6. | A simply supported beam of length 8 m carries point load of 4 KN and 7 KN at distances 3 m and 6 m from the left end. Draw SFD and BMD for the beam. | 10M | CO-2 | BL-4 |
| UNIT-IV |  |  |  |  |
| 7. | A timber beam of rectangular section is to support a uniformly distributed load of 30 k N/m over a span of 4 m when beam is simply supported. If the depth of section is to be twice the breadth, and the stress in the timber is not to exceed 8 N/mm2, find the dimensions of the cross section. | 10M | CO-3 | BL-4 |
| OR |  |  |  |  |
| 8. | A beam is simply supported and carries a uniformly distributed load of 40KN/m run over the whole span. The section of the beam is rectangular having depth as 500mm. If the maximum stress in the material of the beam is 120 N/mm2 and moment of inertia of the section is 7 x 108mm4 , find the span of the beam. | 10M | CO-3 | BL-4 |
| UNIT-V |  |  |  |  |
| 9. | A simply supported beam carries a uniformly distributed load of intensity 30 N/mm over the entire span of 2 m. The cross section of beam is a T-section having flange 125 x 25 mm and web 175 x 25 mm. Calculate the maximum shear stress for the section subjected to maximum shear force. Also draw the shear stress distribution diagram. | 10M | CO-4 | BL-3 |
| OR |  |  |  |  |
| 10. | An I-section beam of flanges 200mm x 20mm and web of 300 mm x 10mm is acted on by a shear force of 150 kN. Determine i) the maximum shear stress in the web ii) The maximum shear stress in the flange. iii) The shearing stress at a layer 60mm below the top of the section. Show the shear stress distribution diagram. | 10M | CO-4 | BL-3 |

***