| Subject Code 22PC1ME203 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year I Semester Regular/Supplementary Examinations, December/January 2025

THERMODYNAMICS

(ME)

Time: 3 hours                                                           		           Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

Data books allowd.

PART-A				5X2=10M

|  | What is path function? | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | Define specific heat at constant volume | 2M | CO-1 | BL-2 |
|  | Mention two examples of steady state processes | 2M | CO-1 | BL-2 |
|  | What are the methods available to examine the quality of steam? | 2M | CO-2 | BL-1 |
|  | Define and specify the empirical relation of Internal Energy. | 2M | CO-3 | BL-2 |

PART-B				5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | a) State and explain Zeroth law of thermodynamics. b) Discuss about macroscopic and microscopic points of view with an example | 5M 5M | CO-1 CO-1 | BL-1 BL-2 |
| OR |  |  |  |  |
| 2 | Determine A piston-cylinder device operates 1kg of fluid at 20atm pressure. The initial volume is 0.04m3. The fluid is allowed to expand reversibly following a process P.V1.4=constant. So that the volume becomes double. The fluid is then cooled at a constant pressure until the piston comes back to the original position keeping the piston unaltered, is added reversibly to restore it to the initial pressure. Calculate the work on in the cycle. | 10M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |
| 3 | State and derive the first law of thermodynamics for a closed system undergoing a cycle. | 10M | CO-1 | BL-1 |
| OR |  |  |  |  |
| 4 | Derive the steady flow energy equation with single stream entering and leaving the control volume. | 10M | CO-1 | BL-3 |
| UNIT-III |  |  |  |  |
| 5 | a) What are the advantages and limitations of Second Law of Thermodynamics? | 3M | CO-1 | BL-1 |
|  | b) A heat pump extracts heat from a cold reservoir at -5°C and delivers it to a hot reservoir at 35°C. If the heat pump's COP is 5, determine the work input required to deliver 3000 kJ of heat. | 7M | CO-1 | BL-3 |
| OR |  |  |  |  |
| 6 | a) Define entropy and explain its physical significance in thermodynamics. b) A copper block of mass 1kg at 500 k is immersed in lake at 300k till it reaches thermal equilibrium with water. Find i) The total heat transfer to the lake ii) Change in entropy of the lake. Iii) Change in entropy of copper (Cp of copper =0.386 kJ/kgK, Cp of water = 4.187 kJ/kgK. | 2M 8M | CO-1 CO-1 | BL-1 BL-3 |
| UNIT-IV |  |  |  |  |
| 7 | A steam turbine operates under steady-state conditions. Steam enters the turbine at 5 MPa, 400°C, and exits at 100 kPa with a dryness fraction of 0.9. If the mass flow rate of steam is 2 kg/s, determine the power output of the turbine. | 10M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 8 | Compare and contrast Rankine and Brayton cycles in terms of efficiency and applications with graphical illustrations. | 10M | CO-2 | BL-4 |
| UNIT-V |  |  |  |  |
| 9 | - A gas mixture consists of 2 moles of oxygen (O₂) and 3 moles of nitrogen (N₂). Calculate the specific heat capacities at constant pressure and constant volume for the mixture. | 7M | CO-3 | BL-3 |
|  | - Explain the concept of partial pressures using Dalton's law. | 3M | CO-3 | BL-2 |
| OR |  |  |  |  |
| 10 | Derive the Maxwell relations and explain their significance in thermodynamics. | 10M | CO-3 | BL-3 |

****