| Subject Code 22PC1EC206 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year II Semester Regular /Supplementary Examinations, May/June 2025

ANALOG AND DIGITAL COMMUNICATIONS

(ECE)

Time: 3 hours                                                                                Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | Sketch the spectral diagram of a single tone DSBSC modulated wave with a message signal of frequency fm and a carrier signal fc frequency. | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | The 𝑇 − 1 carrier system is used to multiplex 24 voice signals based on 8 − 𝑏𝑖𝑡 𝑃𝐶𝑀 word. The bit duration of the time-division multiplexed signal is 𝑇𝑏 = 0.647 𝜇𝑠𝑒𝑐. Calculate the transmission bandwidth required to accommodate the raised cosine spectrum with roll-off factor, 𝛼 = 1. | 2M | CO-3 | BL-3 |
|  | What is Nyquist’s first criterion for zero ISI in baseband transmission? | 2M | CO-5 | BL-2 |
|  | Draw the BPSK waveform for 10010011. | 2M | CO-4 | BL-3 |
|  | What are optimum filter receivers? | 2M | CO-4 | BL-1 |

PART-B				        5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) A double side band suppressed carrier with center frequency of 800 KHz and bandwidth 50 KHz is required to be frequency translated to the range with center frequency of 455 KHz. Design a suitable system for frequency translation and explain your design with necessary block diagram and spectrum. | 5M | CO-1 | BL-4 |
|  | b) Compare the characteristics of Narrow band FM and wide –band FM signals. | 5M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 2. | Explain the generation and detection of AM signal with a circuit diagram and mathematical equations and also draw the spectrum. | 10M | CO-2 | BL-3 |
| UNIT-II |  |  |  |  |
| 3. | - A signal 𝑚(𝑡) of Bandwidth 𝐵 = 4𝑘𝐻𝑍 is transmitted using a binary PCM. Calculate the transmission Bandwidth and the output SNR assuming 10𝑏𝑖𝑡𝑠/𝑠𝑎𝑚𝑝𝑙𝑒 and Nyquist rate sampling. | 5M | CO-3 | BL-4 |
|  | - With relevant equations and block diagrams explain the PCM signal. | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 4. | Explain delta modulation with relevant block diagram and waveforms. Discuss two types of errors in a delta modulation system. | 10M | CO-5 | BL-2 |
| UNIT-III |  |  |  |  |
| 5. | Explain the following with respect to pulse shaping: - Zero ISI - Duobinary pulse - Modified duobinary pulse. | 10M | CO-3 | BL-2 |
| OR |  |  |  |  |
| 6. | Compute the power spectral density of polar line coding format? | 10M | CO-4 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7. | With block diagram explain BPSK modulation and demodulation and calculate it’s bandwidth. | 10M | CO-4 | BL-4 |
| OR |  |  |  |  |
| 8. | Describe the FSK technique with waveforms. Also describe coherent and non-coherent detection of FSK with relevant block diagrams. | 10M | CO-5 | BL-3 |
| UNIT-V |  |  |  |  |
| 9. | Explain the following with respect to correlated receivers: - Baseband signal receiver. - Matched filter receiver | 10M | CO-5 | BL-3 |
| OR |  |  |  |  |
| 10. | Determine the probability of error of a coherent FSK signaling scheme. | 10M | CO-5 | BL-4 |

***